APPENDICES

SAMPLE DETERMINATIONS TO IDENTIFY WATER QUALITY LIMITING LAKES

# Purpose. The purpose of the following is to determine those lakes where a density of one dwelling unit per 150* feet of shoreline of the lake would increase the phosphorus concentration of the lake water by 5 parts per billion or more.

# Method. The maximum change in phosphorus loading will be determined by:

# (1)  L = P x DA x R

where		= change

L	= phosphorus loading

P	= phosphorus concentration

P	= .005 grams/meters3  (approximately equivalent to 5 parts per billion)

R	= run-off coefficient (average annual run-off)

DA	= drainage area

Assuming 1/4 time occupancy, four people per dwelling, and a contribution of .4 x 103 grams of phosphorus per person per year to the body of water, the water quality limitation to shoreline development is:

(2)  max. L	= number of dwelling units

.4 x 103 grams per year

If development was allowed to a density of 1 dwelling unit per 150 feet of shoreline, the maximum number of dwellings allowed according to shoreline length only would be:

# (3)  shoreline length = number of dwelling units

150 feet

If (3) is greater than (2), the lake is a water quality limiting lake;
If (2) is greater than (3), the lake is not a water quality limiting lake;

_________________

*	Since the minimum shore lot frontage on major water bodies is 150 feet measured as a straight line between the points of intersection of side lot lines with the normal high water mark of the shoreline, and due to the irregularity of shoreline, the actual average shoreline measure along the normal high water mark for such lot is estimated to be 150 feet.

Sample 1

Madawaska Lake, Aroostook County

Characteristics:

Drainage area	= 82.9  x  106m2

Run-off coefficient	= .5 m/year

Shoreline length	= 54,000 feet

P	= .005 grams

Calculations:

(1) Maximum L = 20.7  x  104 grams per year.

(2) Number of dwelling units allowed based on water quality limitations = 518 dwelling units.

(3) Number of dwelling units allowed based on shoreline length = 360 dwelling units.

Result: Madawaska Lake is not a water quality limiting lake.

# Sample 2

Tomah Lake, Washington County

Characteristics:

Drainage Area	= 1.8  x  106m2

Run-off coefficient	= .6 m/year

Shoreline length	= 7,000 feet

P	= .005 grams

Calculations:

(1) Maximum L = 5.4  x  103 grams per year.

(2) Number of dwelling units allowed based on water quality limitations = 14 dwelling units.

(3) Number of dwelling units allowed based on shoreline length = 47 dwelling units.

Result: Tomah Lake is a water quality limiting lake.

GUIDELINES FOR VEGETATIVE STABILIZATION

Areas of disturbed soil, including but not limited to areas that are filled, graded or otherwise disturbed during construction projects, should be stabilized according to the following guidelines. These guidelines do not apply to forest management activities and are not strict regulations, and therefore alternative methods of stabilizing soil may be used. However, whenever soil stabilization or stabilization of disturbed areas is required by regulation or by the terms of individual permits, individuals must assure that either these guidelines, or measures equally effective in stabilizing disturbed areas of soil are employed.

The goals to be achieved by proper stabilization are the avoidance of accelerated soil erosion and the avoidance of sedimentation or pollution of water bodies. All stabilization measures must be maintained so that grass or other vegetation remains intact and healthy, otherwise these measures will be ineffective.

In general:

Sterile soils such as sands and gravels should be covered with 2 to 4 inches of soil medium that will support vegetative growth.

Disturbed soil areas should be graded such that runoff water is either minimized or eliminated from running over the site.

Disturbed areas which can be seeded between May 1 and September 15 should be prepared and seeded during that period.

Disturbed areas which cannot be seeded between May 1 and September 15 should be mulched with hay, straw or some other suitable material to keep them as stable as possible over the winter, and particularly during spring runoff the following year. For over-wintering, mulch must be tacked down, as it is easily blown around on frozen ground, leaving areas of soil exposed. Mulch hay should be applied at a depth of 4 inches, or between 150 to 200 lbs. per 1,000 square feet, over the disturbed site. Mulched over-wintered areas should be prepared and seeded the following spring as soon as conditions allow.

It is not recommended that disturbed areas be seeded after September 15th (“dormant seeding”) for a number of reasons. Among the reasons, seeding rates are doubled, which is more expensive; timing is critical to ensure that germination does not occur before the following spring; there is an increased risk of sedimentation because sites are generally wetter in the fall; the thicker mulch must be removed in the spring in order to allow the germinating seed to survive; and the application of fertilizer during this time increases the risk of leaching or runoff loss of nutrients into water bodies.

Seeding preparation, in addition to providing a soil medium that will support vegetative growth if the site is sterile, includes the application of lime and fertilizer, which should be lightly raked prior to seeding. After the area is seeded, it should be lightly watered and then mulched with 70 to 90 lbs. (2 standard bales) per 1,000 square feet of weed free hay or straw to protect the seed. Keep the site stable and moist, and allow the seed to germinate and grow.

For accurate liming as well as fertilization, it is recommended that you have the soil analyzed to determine the specific nutrient requirements of your site.

Lime should be applied at a rate of approximately 140 pounds to 1,000 square feet of area. This rate may vary depending on the natural conditions of the soil on the site. 10-5-20 fertilizer should be applied at a rate of 18.5 lbs. per 1,000 square feet of area. Following the establishment of vegetation, non-phosphorous fertilizer should be used in accordance with the Maine Department of Environmental Protection’s recommendations.

In shoreland areas in particular, fertilizers should be of the "quick release" low phosphorus type, such as 12-4-8 mixtures applied at a rate of 8 pounds per 1,000 square feet of area. If you are near water bodies, it is important not to apply more than approximately this amount of fertilizer, as excess may be washed into streams or lakes and contribute to lowering water quality and such things as algae blooms in lakes.

Following the establishment of vegetation, non-phosphorous fertilizer should be used in accordance with the Maine Department of Environmental Protection’s recommendations.

Fertilizers should never be applied right before thunder storms or before spring runoff, because the great amounts of water running over the land will wash the fertilizer, particularly phosphorus, into water bodies. However, a light watering after the fertilizer is applied will help bind the phosphorus to the soil.

There are many combinations of grasses that can be used. One combination particularly good for providing soil stability, generally referred to as the Soil Conservation Mixture, consists of:  (Proportions, by weight)

Creeping Red Fescue	35%	Kentucky Bluegrass	25%

Annual Rye Grass	15%	Perennial Rye Grass	10%

Red Top	10%	White Dutch Clover	5%

* Oats - See Below

This seed would be applied at a rate of 1 pound per 1,000 square feet. These particular grasses do best if mowed no closer than 2-1/2 to 3 inches from the ground. Of course, other seed mixtures are available.

It is important, in choosing a mixture, to choose one suitable for the site being stabilized. There are many different types of seeding mixtures designed for particular site conditions such as shade, sun, and drainage. Any mix should contain some seed which germinates rapidly to provide the quickest stabilization possible while awaiting the germination of the remaining types.

(*)	For quick germination, oats are very good. They germinate in 7 to 10 days. They should be planted at a rate of approximately 1 to 1-1/2 bushels per acre, in addition to the basic grass mixture. Oats should be mowed when they reach knee height to allow the germinating grasses to receive sunlight.

Alternatives:

As indicated above, other stabilization programs may be used, provided they are equivalently effective in stabilizing disturbed areas and preventing accelerated soil erosion and sedimentation of water bodies. Further assistance may be obtained, including in some cases site-specific recommendations, as follows:

- Local Soil and Water Conservation Districts

- The USDA Natural Resource Conservation Service

- Maine Department of Environmental Protection, Lakes Program

- Landscaping Professionals

- Reputable Lawn and Garden Supply Dealers

The following documents may provide valuable assistance to those developing a soil stabilization plan:

Maine Erosion and Sediment Control Handbook for Construction:  Best Management Practices (Cumberland County Soil & Water Conservation District and Maine Department of Environmental Protection, 1991)

Strategy for Managing Nonpoint Source Pollution From Agricultural Sources and Best Management Guidelines (NPS Agricultural Task Force, 1991)

Erosion and Sediment Control Handbook for Maine Timber Harvesting Operations, Best Management Practices (Maine Forest Service, 1991)

ALPHABETICAL LIST OF LAKES SHOWING WILDLANDS LAKE ASSESSMENT FINDINGS

# LEGEND TO APPENDIX C:

# Resource Ratings:

FSH	= Fisheries	O	= Outstanding

WLD	= Wildlife*	S	= Significant

SC	= Scenic	P	= Present

SH	= Shore Character	m	= missing information

BOT	= Botanic	+,-	= resource needing further field checking due to

public comment

CLT	= Cultural

PHY	= Physical	(+ = positive comment;  - = negative comment)

*An "*" after the wildlife rating indicates an outstanding wildlife value due to an especially concentrated and diverse wildlife value -- one of the criteria for a Management Class 2 Lake.

# Resource Class:

1A	=	lakes of statewide significance with two or more outstanding values

1B	=	lakes of statewide significance with one outstanding value

2	=	lakes of regional significance (with no outstanding values but at least one significant resource value)

3	=	lakes of local or unknown significance (either had no significant or outstanding natural value or information was inadequate to make a determination)

# Land Use:

INAC	=	Relatively Inaccessible--has no road passable with a 2-wheel drive car within approximately 1/4 mile of the lake shore

AC	=	Relatively Accessible

UNDEV	=	Relatively Undeveloped--has less than one development unit per shore mile, taken as an average over the entire lake (a development unit is defined as a single residence, small sporting camp cluster, or similar development)

DEV	=	Relatively Developed

# Management Class:

1 = least accessible, undeveloped high value lakes

1/6 = meets criteria for Mgnt Class 1, but adequately protected by Mgnt Class 6 zoning (remote ponds)

# 2 = accessible, undeveloped lakes with exceptional values

3 = lakes potentially suitable for development

4 = high value, developed lakes

5 = heavily developed lakes

6 = remote ponds

| ALPHABETICAL LIST OF LAKES SHOWING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WILDLANDS LAKE ASSESSMENT FINDINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRINCIPAL LAKE NAME | LAKE # | TOWN NAME | IF&W | SIZE | RESOURCE RATINGS |  |  |  |  |  |  | RESOURCE | LAND USE |  | MGNT |
|  |  |  | REG | (AC) | FSH | WLD | SC | SH | BOT | CLT | PHY | CLASS | ACCESS | DEV | CLASS |
| ABBIE P | 3360 | BOWMANTOWN TWP | D | 12 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| ABOL DEADWATER | 2058 | T02 R10 WELS | F | 150 | O |  |  |  |  |  | S | 1B | AC | UNDEV |  |
| ACKLEY P | 2200 | MOUNT CHASE | F | 19 |  |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| ALDER L | 1778 | T11 R04 WELS | G | 160 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ALDER P | 0120 | T03 R05 BKP WKR | D | 108 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ALDER P | 2504 | ALDER BROOK TWP | E | 37 | S |  | S |  |  |  | S | 2 | AC | UNDEV |  |
| ALLAGASH L | 9787 | T08 R14 WELS | E | 4260 | O | O | O | O |  | S | O | 1A | AC | UNDEV | 2 |
| ALLAGASH P | 2970 | T09 R15 WELS | G | 89 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| ALLEN P | 4516 | T35 MD | C | 83 | m | S | S- |  |  |  |  | 2 | INAC | DEV |  |
| ALLEN P (LOWER) | 4504 | T34 MD | C | 51 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ALLEN P (MIDDLE) | 4502 | T34 MD | C | 32 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ALLEN P (UPPER) | 4500 | T34 MD | C | 44 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ALLIGATOR L | 4498 | T34 MD | C | 1159 | O |  | O | S |  |  |  | 1A | AC | UNDEV | 2 |
| ALLIGATOR P | 0502 | TA R11 WELS | E | 47 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| AMBEJEJUS L* | PAMB | T01 R09 WELS | F | 3289 | S |  | S | S |  | O | S | 1A | AC | DEV | 3,5 |
| ANDERSON P | 4426 | T10 SD | C | 16 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ARNOLD P | 3332 | COBURN GORE | D | 148 | S |  | O |  |  | O |  | 1A | AC | DEV | 4 |
| ATKINS P | 4176 | T08 R09 WELS | G | 32 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ATTEAN P | 2682 | ATTEAN TWP | E | 2745 | O |  | O | O | O |  | O | 1A | AC | UNDEV | 2 |
| ATWOOD P | 4250 | T05 R08 WELS | F | 50 | S |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| AUSTIN P | 0278 | BALD MTN TWP T2R3 | D | 684 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| AUSTIN P | 2410 | T05 R17 WELS | E | 43 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| AUSTIN P (LITTLE) | 0244 | BALD MTN TWP T2R3 | D | 110 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| AVERY P | 3001 | T07 R15 WELS | E | 60 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| AZISCOHOS L* | 3290 | LINCOLN PLT | D | 6700 | O | O | S | S |  | O | S | 1A | AC | DEV |  |
| AZISCOHOS L (NORTH)* | AZ02 | PARKERTOWN TWP | D | 4700 | O | O | S | S |  | O | S | 1A | AC | DEV |  |
| AZISCOHOS L (SOUTH)* | AZ01 | LINCOLN PLT | D | 2000 | O | O | S | S |  | O | S | 1A | AC | DEV | 3 |
| AZISCOHOS P | 3106 | MAGALLOWAY TWP | D | 12 | S |  |  |  |  |  |  | 1B | INAC | UNDEV | 6 |
| B LAKE | 1718 | HAMMOND | G | 66 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| B POND | 0478 | TB R11 WELS | F | 644 | O |  | S |  |  |  | S | 1B | AC | UNDEV |  |
| BAIT P | 0978 | T04 INDIAN PURCHASE TWP | F | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BAKER FLOWAGE | 0272 | MAYFIELD TWP | D | 40 |  | S |  |  |  |  | S | 2 | INAC | UNDEV |  |
| BAKER L | 2400 | T07 R17 WELS | E | 1231 | S |  |  |  |  | S | S | 2 | AC | UNDEV |  |
| BAKER P | 0422 | BOWDOIN COL GR WEST | E | 10 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BAKER P | 4060 | MOXIE GORE | D | 93 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BAKER P | 4122 | TOMHEGAN TWP | E | 74 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BAKER P | 5110 | HAYNESTOWN TWP | E | 270 | O |  | S |  |  |  |  | 1B | AC | UNDEV |  |
| BAKER STREAM P | 7104 | BALD MTN TWP T2R3 | D | 12 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| BALD MOUNTAIN P | 0314 | BALD MTN TWP T2R3 | D | 1152 | O | O | O | O |  |  |  | 1A | AC | UNDEV | 2 |
| BARBLESS P | 4384 | T07 SD | C | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BARKER P | 3118 | BOWMANTOWN TWP | D | 35 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BARREN P | 1220 | T24 MD BPP | C | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BARRETT P | 2658 | HOLEB TWP | E | 34 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BARTLETT P | 1986 | T10 R09 WELS | G | 77 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| BARTLEY P (BIG) | 2656 | HOLEB TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BARTLEY P (LITTLE) | 2664 | HOLEB TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BASIN P | 0042 | PIERCE POND TWP | D | 80 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BASKAHEGAN L | 1078 | BROOKTON TWP | F | 6944 | S | O |  |  |  | S | S | 1B | AC | UNDEV |  |
| BAY P (WEST) | 4396 | T07 SD | C | 249 |  | O |  |  |  |  |  | 1B | INAC | UNDEV | 1 |
| BEAN P | 0656 | T02 R12 WELS | E | 16 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BEAN P | 1880 | T11 R17 WELS | G | 44 |  |  |  |  |  |  | P | 3 | AC | UNDEV |  |
| BEAN P (LOWER) | 0646 | RAINBOW TWP | E | 37 | S | S |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BEAN P (MIDDLE) | 0648 | RAINBOW TWP | E | 10 |  | S |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BEAN P (UPPER) | 0650 | RAINBOW TWP | E | 25 | S | S |  |  |  |  | S | 2 | INAC | UNDEV | 6 |
| BEAN POT P | 4026 | T05 R15 WELS | E | 52 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEANS (BEAN) P | 0204 | PLEASANT RIDGE PLT | D | 20 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BEAR BROOK BOG | 4020 | T06 R15 WELS | E | 15 | m | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BEAR P | 0528 | TA R11 WELS | E | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEAR P | 0636 | RAINBOW TWP | E | 30 | S |  |  |  |  |  | S | 2 | INAC | UNDEV | 6 |
| BEAR P | 0882 | ELLIOTTSVILLE TWP | E | 17 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BEAR P | 2136 | CEDAR LAKE TWP | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BEAR P | 2934 | T02 R13 WELS | E | 81 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BEAR P | 4018 | T06 R15 WELS | E | 138 |  | S | O |  |  |  |  | 1B | INAC | UNDEV | 6 |
| BEAR P | 4480 | T28 MD | C | 19 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEATTIE P | 5066 | BEATTIE TWP | E | 27 | S |  |  |  |  |  |  | 2 | INAC | DEV | 6 |
| BEAU L | 9785 | T19 R11 WELS | G | 2003 | S |  |  |  | S | S | S | 1B | AC | UNDEV | 3 |
| BEAVER MOUNTAIN L | 3562 | SANDY RIVER PLT | D | 543 | S |  | S |  |  |  |  | 2 | AC | DEV | 5 |
| BEAVER P | 0484 | SHAWTOWN TWP | E | 27 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BEAVER P | 0670 | T03 R11 WELS | E | 15 | m |  | S | S |  |  |  | 2 | INAC | UNDEV | 6 |
| BEAVER P | 1662 | T14 R05 WELS | G | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEAVER P | 1872 | T12 R17 WELS | G | 70 |  |  |  |  |  |  | P | 3 | AC | UNDEV |  |
| BEAVER P | 2636 | FORSYTH TWP | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEAVER P | 3076 | T08 R05 WELS | G | 67 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEAVER P | 3310 | MAGALLOWAY TWP | D | 179 | O |  | O- |  |  |  |  | 1A | AC | UNDEV | 2 |
| BEAVER P | 3354 | SEVEN PONDS TWP | D | 20 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| BEAVER P | 3588 | TOWNSHIP D | D | 20 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BEAVER P | 7309 | T42 MD BPP | C | 20 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BEAVER P | 8739 | RANGELEY PLT | D | 14 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEAVER P | 9756 | ALDER BROOK TWP | E | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEAVER P (BIG) | 0610 | RAINBOW TWP | E | 45 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BEAVER P (BIG) | 4162 | T07 R09 WELS | F | 329 | S | + | S | + |  |  | + | 2 | INAC | UNDEV |  |
| BEAVER P (LITTLE) | 0612 | T03 R11 WELS | E | 10 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BEAVER P (LITTLE) | 3312 | MAGALLOWAY TWP | D | 50 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| BEAVER P (LITTLE) | 4164 | T07 R09 WELS | G | 122 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BEAVER P (LITTLE) | 9700 | RAINBOW TWP | E | 8 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BEAVER TAIL P | 1536 | T14 R09 WELS | G | 128 | S | S+ |  |  |  |  |  | 2 | AC | UNDEV |  |
| BEAVERDAM L (NORTH) | 1312 | T26 ED BPP | C | 147 | S |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BEAVERDAM L (SOUTH) | 1232 | T26 ED BPP | C | 160 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BECK P | 5142 | T03 R05 BKP WKR | D | 32 | S |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| BEN L | 1638 | WALLAGRASS | G | 37 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BENJAMIN P | 2684 | ATTEAN TWP | E | 121 | m |  | S+ | S+ |  |  |  | 2 | INAC | UNDEV | 6 |
| BENSON P (BIG) | 0864 | T07 R09 NWP | E | 320 | O |  | O+ |  |  | S |  | 1B | AC | UNDEV | 2 |
| BERRY P | 2550 | JOHNSON MOUNTAIN TWP | E | 35 |  |  |  |  | S |  |  | 2 | INAC | UNDEV |  |
| BERRY P | 2930 | T02 R13 WELS | E | 71 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BERRY P (LITTLE) | 2554 | JOHNSON MOUNTAIN TWP | E | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BERRY P (LITTLE) | 2932 | T02 R13 WELS | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BIG BOG | 2412 | T05 R17 WELS | E | 1064 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BIG BROOK L | 1480 | T14 R10 WELS | G | 60 |  |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BIG L | 1288 | BIG LAKE TWP | C | 10305 | O | O | + |  | O | O |  | 1A | AC | DEV | 3 |
| BIGELOW P | 3318 | MAGALLOWAY TWP | D | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BILL MORRIS P | 0128 | T03 R05 BKP WKR | D | 23 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BILLINGS P | 1970 | T11 R09 WELS | G | 64 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BILLINGS P #1 | 3108 | PARMACHENEE TWP | D | 20 |  |  |  |  |  | S- |  | 2 | INAC | UNDEV |  |
| BILLINGS P #2 | 3110 | PARMACHENEE TWP | D | 10 |  |  |  |  |  | S- |  | 2 | INAC | UNDEV |  |
| BIRCH RIDGE P #1 | 0514 | TA R11 WELS | E | 11 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BISHOP P | 1572 | T13 R07 WELS | G | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BLACK BROOK DWTR | 9706 | T02 R12 WELS | E | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BLACK BROOK L (BIG) | 1860 | CASWELL | G | 62 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BLACK BROOK L (LIT) | 1862 | CASWELL | G | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BLACK BROOK P | 0040 | PIERCE POND TWP | D | 47 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BLACK BROOK P | 4062 | MOXIE GORE | D | 333 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| BLACK BROOK P #1 | 1184 | T19 MD BPP | C | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BLACK BROOK P #6 | 1192 | T25 MD BPP | C | 19 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BLACK CAT P | 3086 | T08 R06 WELS | G | 27 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BLACK L | 1506 | T15 R09 WELS | G | 147 | O |  | S |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| BLACK L | 1568 | T16 R09 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BLACK P | 0454 | T01 R12 WELS | E | 127 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BLACK P | 1622 | ST JOHN PLT | G | 21 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BLACK P | 2896 | T06 R14 WELS | E | 1450 | m |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| BLACK P (LITTLE NO) | 1508 | T15 R09 WELS | G | 6 | S |  | S |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BLACK P (LITTLE S0) | 1510 | T15 R09 WELS | G | 7 | S |  | S |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BLACK P (LOWER) | 7724 | OXBOW TWP | D | 30 | O |  |  |  |  |  |  | 1B | AC | DEV |  |
| BLACK P (UPPER) | 3362 | BOWMANTOWN TWP | D | 30 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BLACK SPRUCE P | 2844 | T08 R10 WELS | G | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BLAKE L | 1648 | T16 R06 WELS | G | 128 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BLAKESLEE L | 5114 | HAYNESTOWN TWP | E | 55 | O |  | S |  |  |  |  | 1B | AC | UNDEV |  |
| BLANCHARD P | 5056 | ALDER STREAM TWP | D | 9 |  |  |  |  |  |  | O | 1B | INAC | UNDEV |  |
| BLANCHARD P #1 | 2366 | LANG TWP | D | 4 |  |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| BLOOD (DUCK) P | 2928 | T02 R13 WELS | E | 48 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BLOOD L | 1458 | T14 R16 WELS | G | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BLUE P | 1468 | T13 R13 WELS | G | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BLUFF P | 0434 | FRENCHTOWN TWP | E | 10 | S |  |  |  |  |  | S | 2 | INAC | UNDEV | 6 |
| BLUFFER P | 2794 | T08 R11 WELS | G | 40 | S | + | + |  | + |  | + | 2 | INAC | UNDEV |  |
| BLUFFER P (LITTLE) | 2796 | T08 R11 WELS | G | 12 | S | + |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BLUFFER P (UPPER) | 2798 | T08 R11 WELS | G | 15 | S | + |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BOARDWAY P (BIG) | 0494 | TA R11 WELS | E | 15 | S |  |  |  |  |  | S | 2 | INAC | UNDEV | 6 |
| BOARDWAY P (LITTLE) | 0496 | TA R11 WELS | E | 10 |  |  |  |  |  |  | S | 2 | INAC | UNDEV |  |
| BOBS P | 2426 | T04 R17 WELS | E | 27 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BOG L | 1376 | MARION TWP | C | 25 |  | S |  |  |  |  | O | 1B | INAC | UNDEV |  |
| BOG L | 1664 | T14 R05 WELS | G | 28 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| BOG P | 0530 | TA R11 WELS | E | 40 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BOG P | 0944 | CEDAR LAKE TWP | F | 44 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BOG P | 2680 | ATTEAN TWP | E | 95 |  |  |  | + | + |  | S | 2 | INAC | UNDEV |  |
| BOG P | 2864 | T07 R12 WELS | E | 35 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BOGUS MEADOW P | 4380 | T07 SD | C | 26 | S | O | S |  |  |  |  | 1B | INAC | UNDEV | 1 |
| BOODY P | 3000 | T08 R08 WELS | G | 28 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BOOT P | 0816 | T07 R09 NWP | E | 17 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BOTTLE L | 4702 | LAKEVILLE | F | 281 | S |  | S |  |  |  |  | 2 | AC | DEV | 5 |
| BOTTLE P | 2070 | T02 R09 WELS | F | 90 |  |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| BOULDER P | 2672 | T05 R07 BKP WKR | E | 30 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BOUNDARY P | 5070 | BEATTIE TWP | E | 70 | S |  |  |  | S |  |  | 2 | AC | UNDEV |  |
| BOUNDARY P (SOUTH) | 3346 | MASSACHUSETTS GORE | D | 10 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BOWLES L | 1154 | DAY BLOCK TWP | C | 64 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| BOWLIN P | 2188 | T05 R08 WELS | F | 115 | S |  | S |  |  |  |  | 2 | AC | UNDEV | 3 |
| BOWLIN P (LITTLE) | 2194 | T05 R07 WELS | F | 34 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| BOYD L | 2158 | ORNEVILLE TWP | F | 1005 | S |  |  |  |  | S | O | 1B | AC | DEV | 5 |
| BRACEY P | 4508 | T34 MD | C | 14 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BRACKETT P | 0290 | BLANCHARD TWP | E | 10 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BRADFORD P | 3651 | MORO PLT | G | 12 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BRALEY L | 1696 | T03 R04 WELS | F | 147 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BRANCH L (EAST) | 2130 | CEDAR LAKE TWP | F | 1100 |  |  |  |  |  |  | S | 2 | AC | UNDEV |  |
| BRANCH L (SOUTH) | 2144 | SEBOEIS PLT | F | 2035 | S | S |  |  |  | S |  | 2 | AC | DEV |  |
| BRANCH P (1ST WEST) | 0440 | SHAWTOWN TWP | E | 119 | O |  | + |  |  |  |  | 1B | AC | DEV |  |
| BRANCH P (2ND&3RD W) | 0442 | SHAWTOWN TWP | E | 214 | O |  | S+ |  |  |  |  | 1B | AC | UNDEV |  |
| BRANCH P (EAST) | 2822 | T07 R11 WELS | E | 45 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BRANCH P (MIDDLE) | 0912 | EBEEMEE TWP | F | 34 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| BRANDY P | 1704 | WEBBERTOWN TWP | G | 28 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BRANDY P | 2898 | CHESUNCOOK TWP | E | 650 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BRANDY P | 9651 | T39 MD | F | 723 | S | O |  |  |  |  | S | 1B | AC | DEV | 3 |
| BRASSUA L | 4120 | ROCKWOOD STRIP T01 R01 | E | 8979 | S |  |  |  |  | O |  | 1B | AC | DEV | 3 |
| BRAYLEY P | 2706 | T07 R10 WELS | F | 6 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BROKEN BRIDGE P | 3264 | ALBANY TWP | A | 20 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BROWN P | 0788 | BOWDOIN COL GR WEST | E | 18 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BROWN P | 4178 | T08 R09 WELS | G | 104 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| BRULEAU P | 1456 | T14 R15 WELS | G | 35 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BUCK P | 0644 | RAINBOW TWP | E | 6 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| BUCKLEY P | 2840 | T08 R10 WELS | G | 18 | S | + | + |  |  |  |  | 2 | INAC | UNDEV |  |
| BURNHAM P | 0392 | BIG MOOSE TWP | E | 426 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| BURNT P | 1996 | T09 R07 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| BURNTLAND L | 4792 | T35 MD | C | 80 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| BURNTLAND P | 1878 | T12 R17 WELS | G | 70 |  |  |  |  |  |  | P | 3 | INAC | UNDEV |  |
| BURNTLAND P | 2624 | DENNISTOWN PLT | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BUTLER P | 0056 | LEXINGTON TWP | D | 28 |  |  |  |  |  | S |  | 2 | INAC | UNDEV |  |
| BUTLER P | 0166 | FLAGSTAFF TWP | D | 18 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| BUTLER P | 5132 | KING & BARTLETT TWP | D | 45 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| BUTTERFIELD L | 1848 | CASWELL | G | 13 | S |  |  |  | O |  |  | 1B | AC | UNDEV |  |
| C POND | 3278 | C SURPLUS | D | 173 | S | S | O | S |  |  | + | 1B | INAC | DEV |  |
| CALL P | 0140 | LOWER ENCHANTED TWP | D | 16 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CAMERON BOG | 9511 | HAMMOND | G | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CAMP P | 0822 | T07 R09 NWP | E | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CAMPBELL L | 4794 | T35 MD | C | 35 | m |  |  |  |  | S- |  | 2 | AC | DEV |  |
| CAMPBELL P | 2574 | BLAKE GORE | E | 15 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| CANADA FALLS L | 2516 | PITTSTON ACAD GRANT | E | 2627 | S | S |  |  |  | S | O | 1B | AC | UNDEV |  |
| CAPE HORN P | 2568 | PRENTISS TWP T04 R04 NBKP | E | 22 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| CAREY L | 3014 | T08 R06 WELS | G | 93 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CARIBOU L | 1692 | T03 R04 WELS | F | 256 |  |  |  |  |  | S |  | 2 | INAC | UNDEV |  |
| CARIBOU L* | CHCA | T02 R12 WELS | E | 4600 | O | O |  |  | O | O | O | 1A | AC | DEV | 3 |
| CARIBOU P | 0176 | MT ABRAM TWP | D | 10 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CARIBOU P | 1976 | T11 R09 WELS | G | 12 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| CARIBOU P (BIG) | 4142 | T07 R10 WELS | F | 64 | S |  | S | S | O |  |  | 1B | INAC | UNDEV | 1 |
| CARIBOU P (LITTLE) | 4144 | T07 R10 WELS | G | 10 |  |  | + |  |  |  |  | 3 | INAC | UNDEV |  |
| CARLISLE P | 1766 | T08 R03 WELS | G | 21 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CARLOE P | 9656 | T26 ED BPP | C | 20 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CARPENTER P | 0275 | T07 R11 WELS | E | 160 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CARR P | 1598 | T13 R08 WELS | G | 307 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| CARRY P | 0678 | T03 R11 WELS | E | 17 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| CARRY P | 1684 | MADAWASKA LAKE TWP | G | 64 | S |  |  |  |  | S |  | 2 | INAC | UNDEV |  |
| CARRY P | 3016 | T08 R07 WELS | G | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CARRY P (EAST) | 0044 | CARRYING PLC TWN TWP | D | 267 | O |  |  |  |  | S |  | 1B | AC | DEV |  |
| CARRY P (MIDDLE) | 0046 | CARRYING PLC TWN TWP | D | 126 | S |  |  |  |  | S |  | 2 | AC | DEV |  |
| CARRY P (WEST) | 0048 | CARRYING PLC TWN TWP | D | 675 | O |  |  |  |  | O |  | 1A | AC | DEV | 4 |
| CASSIDY DEADWATER | 2944 | T04 R15 WELS | E | 236 |  | S |  |  |  |  | P | 2 | AC | UNDEV |  |
| CATHANCE L | 9661 | CATHANCE TWP | C | 2905 | O | O |  |  |  | S | O | 1A | AC | DEV | 4 |
| CATHANCE L (LITTLE) | 1382 | CATHANCE TWP | C | 140 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CAUCOMGOMOC L | 4012 | T06 R14 WELS | E | 5081 | O | O* | S | S |  | S | O | 1A | AC | UNDEV | 2 |
| CEDAR L | 2004 | CEDAR LAKE TWP | F | 685 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| CEDAR P | 0474 | TB R10 WELS | F | 65 | O |  |  |  |  |  | S | 1B | INAC | UNDEV | 1/6 |
| CEDAR P | 2654 | HOLEB TWP | E | 5 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| CEDAR P | 2846 | T08 R10 WELS | G | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CENTER P | 1582 | T10 R08 WELS | G | 128 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| CENTER P | 4040 | SOLDIERTOWN T02 R03 NBKP | E | 51 | S | + |  |  |  | S |  | 2 | INAC | DEV |  |
| CHAIN L (FIRST) | 1236 | T26 ED BPP | C | 336 | O |  |  |  |  | S | O | 1A | AC | DEV | 4 |
| CHAIN L (LOWER) | 4732 | SAKOM TWP | C | 179 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| CHAIN L (MIDDLE) | 4734 | OQITON TWP | F | 221 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| CHAIN L (SECOND) | 1234 | T26 ED BPP | C | 589 | O |  |  |  |  | S | O | 1A | AC | DEV | 4 |
| CHAIN L (THIRD) | 1314 | T26 ED BPP | C | 157 | S |  |  |  |  |  |  | 2 | INAC | DEV |  |
| CHAIN L (UPPER) | 4736 | OQITON TWP | F | 717 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CHAIN OF PONDS | 5064 | CHAIN OF PONDS TWP | D | 700 | O | O | O | S |  | S | O | 1A | AC | UNDEV | 2 |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHAIRBACK P (EAST) | 0802 | T07 R09 NWP | E | 46 | S |  |  |  |  |  | S | 2 | INAC | UNDEV | 6 |
| CHAIRBACK P (WEST) | 0796 | T07 R09 NWP | E | 47 | O |  |  |  |  |  | S | 1B | INAC | UNDEV | 1/6 |
| CHALK P | 3270 | ALBANY TWP | A | 25 | m |  |  |  |  |  |  | 3 | AC | DEV |  |
| CHALK P | 4478 | T22 MD | C | 32 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CHAMBERLAIN L | 2882 | T07 R12 WELS | E | 11084 | O | O |  |  |  | O |  | 1A | AC | UNDEV |  |
| CHAMBERLAIN P | 3026 | T07 R06 WELS | G | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CHANDLER DEADWATER | 9179 | T09 R08 WELS | G | 14 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CHANDLER L | 1994 | T09 R08 WELS | G | 401 | S |  | O | S |  |  |  | 1B | AC | UNDEV |  |
| CHANDLER P | 2834 | T08 R10 WELS | G | 115 | S |  | S |  |  | + |  | 2 | AC | UNDEV |  |
| CHARLES (CHARLIE) P | 1454 | T14 R15 WELS | G | 60 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CHASE L | 2752 | T09 R10 WELS | G | 403 | S |  | S | + |  | O |  | 1B | AC | UNDEV |  |
| CHASE P | 3632 | MORO PLT | G | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CHASE P | 5050 | JIM POND TWP | D | 14 |  | S |  |  |  |  |  | 2 | AC | DEV |  |
| CHASE P (FIRST) | 1538 | T14 R09 WELS | G | 12 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| CHASE P (SECOND) | 1540 | T14 R09 WELS | G | 182 | S |  | S |  |  |  | S | 2 | INAC | UNDEV |  |
| CHASE P (THIRD) | 1542 | T14 R09 WELS | G | 102 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CHASE STREAM P | 4080 | CHASE STREAM TWP | D | 75 |  |  | S |  |  |  |  | 2 | AC | DEV |  |
| CHASE STREAM P | 4093 | MISERY TWP | E | 31 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| CHASE STREAM P (LIT) | 5798 | MISERY TWP | E | 17 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CHENEY P | 2494 | HAMMOND TWP | E | 99 | S |  |  |  |  |  | S | 2 | AC | UNDEV | 3 |
| CHESUNCOOK L* | 0662 | T03 R12 WELS | E | 23070 | O | O* | + |  | O | O | O | 1A | AC | UNDEV |  |
| CHESUNCOOK L* | CHCH | T03 R12 WELS |  | 18470 | O | O* | + |  | O | O | O | 1A | AC | UNDEV | 2 |
| CHESUNCOOK P | 0672 | T03 R11 WELS | E | 272 | S |  | O | O |  |  | O | 1A | INAC | UNDEV | 6 |
| CHUB P | 5100 | HOBBSTOWN TWP | E | 24 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| CHURCH P | 0538 | TA R10 WELS | F | 53 | S |  | + |  |  |  |  | 2 | AC | UNDEV |  |
| CHURCHILL L | 2856 | T09 R12 WELS | G | 2923 | O | O* |  |  |  | S | S | 1A | AC | UNDEV | 2 |
| CHURCHILL L (LITTLE) | 9806 | T10 R13 WELS | G | 25 |  |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| CLARKSON P | 2854 | T09 R13 WELS | G | 38 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CLAYTON L | 1882 | CLAYTON LAKE TWP | G | 166 | S |  |  |  |  |  | P | 2 | AC | UNDEV |  |
| CLAYTON L | 1958 | T12 R08 WELS | G | 264 | S |  |  |  |  |  |  | 2 | AC | UNDEV | 3 |
| CLAYTON P | 2406 | T06 R17 WELS | E | 75 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 3 |
| CLEAR L | 1938 | T10 R11 WELS | G | 614 | O |  | O | S |  |  |  | 1A | AC | UNDEV | 2 |
| CLEAR P | 5074 | LOWELLTOWN TWP | E | 21 | m+ |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| CLEARWATER P | 2476 | PRENTISS TWP T04 R04 NBKP | E | 11 |  |  |  |  |  |  | P | 3 | INAC | UNDEV | 6 |
| CLEARWATER P | 2692 | ATTEAN TWP | E | 34 |  |  | + | + |  | O- |  | 1B | INAC | UNDEV | 1/6 |
| CLIFF L | 2780 | T09 R12 WELS | G | 563 | O |  | O | S |  |  | + | 1A | AC | UNDEV | 2 |
| CLIFFORD L | 1304 | GREENLAW CHOPPING TWP | C | 954 | O | O* | + |  |  |  |  | 1A | AC | UNDEV | 2 |
| CLIFFORD P | 0624 | RAINBOW TWP | E | 17 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| CLISH P | 5158 | T05 R20 WELS | E | 21 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| CLOUD P | 0906 | ELLIOTTSVILLE TWP | E | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| COFFEELOS P | 2712 | T06 R11 WELS | E | 198 | O | + |  |  |  |  |  | 1B | AC | UNDEV |  |
| COFFEELOS P (LITTLE) | 2716 | T06 R11 WELS | E | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| COLD BROOK L | 3050 | T03 R04 WELS | F | 47 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| COLD STREAM P | 2538 | MISERY TWP | E | 205 | O |  | S+ |  |  |  |  | 1B | AC | UNDEV |  |
| COLE P | 4676 | WEBSTER PLT | F | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPASS P | 2072 | T02 R09 WELS | F | 100 |  |  |  |  |  |  | P | 3 | AC | UNDEV |  |
| CONIC L | 7511 | BARING PLT | C | 32 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| COOPER P | 0540 | TA R10 WELS | F | 271 | S |  | + |  |  |  |  | 2 | INAC | UNDEV |  |
| CORNER P | 2998 | T09 R16 WELS | G | 60 |  |  |  |  |  |  | P | 3 | INAC | UNDEV |  |
| COW P | 2376 | LANG TWP | D | 62 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| COW P | 2938 | T01 R13 WELS | E | 61 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRANBERRY L (LOWER) | 1174 | T30 MD BPP | C | 285 | S |  | O- |  |  |  |  | 1B | AC | DEV |  |
| CRANBERRY L (UPPER) | 1176 | T30 MD BPP | C | 134 | S |  | O- |  |  |  |  | 1B | AC | UNDEV |  |
| CRANBERRY P | 2916 | LOBSTER TWP | E | 46 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRANBERRY P | 3066 | SCOPAN TWP | G | 45 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRANBERRY P | 3068 | T09 R05 WELS | G | 96 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRANBERRY P | 3314 | MAGALLOWAY TWP | D | 100 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRANBERRY P | 7509 | BARING PLT | C | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRANBERRY P | 8603 | WYMAN TWP | D | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CRANBERRY P #1 | 3018 | T08 R06 WELS | G | 47 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CRANBERRY P #2 | 3020 | T08 R06 WELS | G | 25 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CRANBERRY P(L,NOTCH) | 0784 | BOWDOIN COL GR WEST | E | 7 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| CRANE MILL BROOK RESERVOIR | 2899 | EDMUNDS TWP | C | 24 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRATER P | 0468 | TB R11 WELS | F | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CRATER P | 0487 | T15 R09 WELS | G | 12 | S |  | O | S |  |  | + | 1B | AC | UNDEV |  |
| CRAWFORD L | 1302 | BIG LAKE TWP | C | 1677 | O | S |  |  |  | S |  | 1B | AC | UNDEV |  |
| CRAWFORD P | 0520 | TA R11 WELS | E | 390 | O |  | + | + |  |  |  | 1B | AC | UNDEV |  |
| CRESCENT P | 0652 | RAINBOW TWP | E | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CRESCENT P | 2964 | T09 R15 WELS | G | 320 | O |  | S | S |  |  | P | 1B | AC | UNDEV |  |
| CROCKER P | 0361 | ALBANY TWP | A | 10 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CROCKER P | 2626 | DENNISTOWN PLT | E | 227 | S |  |  |  |  |  | S | 2 | AC | UNDEV |  |
| CROCKETT P | 0286 | BLANCHARD TWP | E | 12 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CROOKED BROOK L | 7393 | FOREST TWP | F | 58 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CROSBY P | 3330 | COBURN GORE | D | 150 | O | S | O |  |  |  |  | 1A | AC | UNDEV | 2 |
| CROSS L | 1494 | T18 R10 WELS | G | 44 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| CROSS L | 1674 | CROSS LAKE TWP | G | 2515 | S |  |  | + | O | S |  | 1B | AC | DEV | 5 |
| CRYSTAL P | 4778 | T40 MD | F | 19 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CUNLIFFE L | 1890 | T12 R13 WELS | G | 134 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CUNLIFFE P | 1450 | T13 R16 WELS | G | 45 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CUNNINGHAM P | 2518 | PITTSTON ACAD GRANT | E | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CUPSUPTIC L* | MLCU | ADAMSTOWN TWP | D | 2199 | O | O | O | S |  | S- |  | 1A | AC | DEV | 4 |
| CUPSUPTIC P | 7726 | OXBOW TWP | D | 20 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| CURRIER P (FIFTH) | 2770 | T09 R11 WELS | G | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CURRIER P (FIRST) | 2768 | T09 R11 WELS | G | 20 | O |  | S |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| CURRIER P (SECOND) | 2774 | T09 R11 WELS | G | 28 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| CURRIER P (SIXTH) | 2754 | T09 R10 WELS | G | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CUT L | 3022 | T07 R06 WELS | G | 294 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| CUT P | 1706 | DUDLEY TWP | G | 26 |  |  |  |  |  |  |  | 3 | INAC | DEV |  |
| CUXABEXIS L | 2892 | T05 R12 WELS | E | 592 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| DAGGETT P | 4006 | T07 R14 WELS | E | 461 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DAISEY P | 0594 | T02 R10 WELS | F | 11 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| DAVIDSON P | 3060 | HERSEYTOWN TWP | F | 87 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| DAVIS (WAPITI) P | 2196 | T05 R07 WELS | F | 69 | S |  | O |  |  |  |  | 1B | AC | DEV | 5 |
| DAVIS P | 5112 | HAYNESTOWN TWP | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DEAD P | 1180 | T25 MD BPP | C | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| DEAD STREAM P | 4066 | WEST FORKS PLT | D | 67 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| DEADWATER (LOWER) | 0946 | CEDAR LAKE TWP | F | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| DEADWATER P (LOWER) | 1978 | T11 R09 WELS | G | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DEADWATER P (UPPER) | 0794 | BOWDOIN COL GR WEST | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DEADWATER P (UPPER) | 1980 | T11 R10 WELS | G | 38 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| DEADWATER P (UPPER) | 2736 | T10 R11 WELS | G | 35 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| DEASEY PONDS | 2304 | T03 R07 WELS | F | 23 |  |  |  |  |  |  | S | 2 | AC | UNDEV |  |
| DEBOULLIE L | 1512 | T15 R09 WELS | G | 262 | O | O | O | S |  |  | + | 1A | INAC | UNDEV | 1 |
| DEBSCONEAG DEADWATER | 2076 | T02 R10 WELS | F | 500 | O | O |  |  |  |  | S | 1A | AC | UNDEV | 2 |
| DEBSCONEAG L (1ST) | 2060 | T02 R10 WELS | F | 320 | O |  | O | S | O |  | S+ | 1A | INAC | UNDEV | 1 |
| DEBSCONEAG L (2ND) | 0586 | T02 R10 WELS | F | 189 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| DEBSCONEAG L (3RD) | 0584 | T01 R10 WELS | F | 1011 | O |  | O | S |  | S | S | 1A | INAC | UNDEV | 1 |
| DEBSCONEAG L (4TH) | 0582 | T01 R11 WELS | E | 227 | O |  | S+ | S |  |  | S | 1B | AC | DEV |  |
| DEBSCONEAG L (5TH) | 0602 | RAINBOW TWP | E | 83 | S |  | S | S |  |  |  | 2 | INAC | UNDEV |  |
| DEBSCONEAG L (8TH) | 0608 | RAINBOW TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DEBSCONEAG P (6TH) | 0580 | T01 R11 WELS | E | 31 | S |  |  |  |  | S | S | 2 | INAC | UNDEV | 6 |
| DEER L | 4512 | T34 MD | C | 38 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| DEER L | 9587 | CASWELL | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DEER P | 1624 | ST JOHN PLT | G | 14 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| DEER P | 2922 | T03 R13 WELS | E | 181 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| DEER P | 3366 | BOWMANTOWN TWP | D | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| DEER P | 5128 | KING & BARTLETT TWP | D | 30 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| DEMO P | 4114 | ROCKWOOD STRIP T02 R01 | E | 192 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| DENNY P | 1524 | T15 R09 WELS | G | 25 | O |  |  | + |  |  |  | 1B | AC | UNDEV |  |
| DEPOT L | 1448 | T13 R16 WELS | G | 883 |  |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| DESOLATION P | 2996 | T08 R16 WELS | G | 70 | S | S | + |  |  |  | P | 2 | AC | UNDEV |  |
| DICKEY P | 1676 | CROSS LAKE TWP | G | 13 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| DILL P | 2378 | DALLAS PLT | D | 11 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| DINGLEY P | 2458 | T04 R05 NBKP | E | 71 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| DINGLEY P (LITTLE) | 2462 | T04 R05 NBKP | E | 17 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| DINGLEY P (UPPER) | 2464 | T04 R05 NBKP | E | 20 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| DIPPER P | 4042 | PITTSTON ACAD GRANT | E | 13 |  |  |  |  |  | O | S | 1B | INAC | UNDEV | 6 |
| DIXON P | 9911 | PIERCE POND TWP | D | 17 | O |  | + |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| DOLBY P | 2124 | TA R07 WELS | F | 1900 |  | O |  |  |  |  | S | 1B | AC | UNDEV |  |
| DOLE P | 2454 | DOLE BROOK TWP | E | 704 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| DONNELL P | 4412 | T09 SD | C | 1120 | O |  | O | O |  | S |  | 1A | AC | DEV | 4 |
| DORITY P | 2496 | HAMMOND TWP | E | 34 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| DOUGHNUT P | 0616 | RAINBOW TWP | E | 12 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| DOUGLAS P | 5044 | KIBBY TWP | D | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DOWNING P | 4428 | T10 SD | C | 70 |  | O |  |  |  |  |  | 1B | AC | UNDEV |  |
| DRAKE L | 1336 | FOREST TWP | F | 236 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| DUBOIS P | 2478 | PRENTISS TWP T04 R04 NBKP | E | 18 | m |  |  |  |  |  | P | 3 | INAC | UNDEV | 6 |
| DUCK L | 4698 | LAKEVILLE | F | 256 | S |  | S | S |  |  |  | 2 | AC | DEV |  |
| DUCK L | 4746 | OQITON TWP | F | 1222 | O |  | S | S |  |  |  | 1B | AC | UNDEV |  |
| DUCK P | 0257 | T04 R11 WELS | E | 21 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| DUCK P | 1698 | T02 R04 WELS | F | 26 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| DUCK P | 2894 | T05 R12 WELS | E | 470 | m | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| DUCK P (BIG) | 2954 | E MIDDLESEX CANAL GR | E | 79 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| DUCK P (LITTLE) | 2956 | E MIDDLESEX CANAL GR | E | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DUDLEY RIPS P | 9572 | T04 R09 NWP | F | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| DUNCAN P | 2480 | PRENTISS TWP T04 R04 NBKP | E | 143 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| DURGIN P | 9855 | JOHNSON MOUNTAIN TWP | E | 15 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| EAGLE L | 1634 | T16 R06 WELS | G | 5581 | O |  |  |  |  | S |  | 1B | AC | DEV |  |
| EAGLE L | 4514 | T34 MD | C | 260 | m |  |  |  |  | S |  | 2 | AC | DEV |  |
| EAGLE L (BIG) | 2858 | EAGLE LAKE TWP | G | 8288 | O | O* |  |  | O | O | P | 1A | AC | UNDEV | 2 |
| EAGLE P | 3090 | DREW PLT | F | 49 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| EAST L | 1464 | T17 R14 WELS | G | 2551 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| EAST L (LITTLE) | 1466 | T17 R14 WELS | G | 189 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| EBEEMEE L | 0914 | EBEEMEE TWP | F | 940 |  |  | S | S |  | O | S | 1B | AC | DEV | 5 |
| EBEEMEE L (UPPER) | 0966 | T04 R09 NWP | F | 196 |  |  |  |  |  |  | S | 2 | AC | DEV | 3 |
| ECHO L | 2766 | T09 R11 WELS | G | 115 | S |  | S |  |  | S |  | 2 | AC | UNDEV |  |
| ED JONES P | 1886 | T12 R15 WELS | G | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| EDDY P | 3546 | SANDY RIVER PLT | D | 9 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| EGG P | 0666 | T03 R12 WELS | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ELAINE P | 0011 | LOWELLTOWN TWP | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ELBOW L* | PELB | T03 INDIAN PURCHASE TWP | F | 895 | S |  | O | S |  | S |  | 1B | AC | DEV | 3 |
| ELBOW P (LOWER) | 1952 | T10 R10 WELS | G | 36 |  |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| ELBOW P (MIDDLE) | 1950 | T10 R10 WELS | G | 52 | S | S | S |  |  |  |  | 2 | AC | UNDEV |  |
| ELBOW P (UPPER) | 1948 | T10 R11 WELS | G | 88 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ELLIS P | 4086 | CHASE STREAM TWP | D | 85 | O |  |  |  |  |  |  | 1B | AC | DEV |  |
| ELLIS P (LOWER) | 2870 | T07 R14 WELS | E | 77 |  |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| ELLIS P (UPPER) | 2992 | T07 R14 WELS | E | 160 |  |  |  |  |  | S |  | 2 | INAC | UNDEV |  |
| ELM P | 2446 | ELM STREAM TWP | E | 250 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| ELM P (LITTLE) | 2444 | ELM STREAM TWP | E | 45 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ENCHANTED P | 0150 | UPPER ENCHANTED TWP | E | 330 | O | O | O | O |  |  | S+ | 1A | INAC | UNDEV | 1 |
| ENCHANTED P (LITTLE) | 0148 | UPPER ENCHANTED TWP | E | 35 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| ENCHANTED P (LOWER) | 0142 | LOWER ENCHANTED TWP | D | 20 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ENDLESS L | 0942 | CEDAR LAKE TWP | F | 1499 | S |  |  |  |  | S | S | 2 | AC | UNDEV | 3 |
| ENOCH L | 1328 | FOWLER TWP | C | 18 |  |  |  |  |  |  |  | 3 | INAC | DEV |  |
| EVERETT P | 5134 | KING & BARTLETT TWP | D | 20 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| EYELET P | 1910 | T12 R12 WELS | G | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| FALLS P | 1490 | T18 R10 WELS | G | 256 | S | S |  |  |  |  |  | 2 | AC | UNDEV | 3 |
| FARRAR P | 1974 | T11 R10 WELS | G | 51 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| FELKER P | 5120 | KING & BARTLETT TWP | D | 50 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| FEMALE P | 0574 | T01 R12 WELS | E | 102 | S |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| FEMALE P (LITTLE) | 0578 | T01 R12 WELS | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FERGUSON P | 1592 | T14 R08 WELS | G | 51 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| FERNALD P | 8868 | PARLIN POND TWP | E | 7 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| FIRST L | 1152 | T37 MD BPP | C | 217 | m | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| FISH P | 0096 | PIERCE POND TWP | D | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| FISH P | 2524 | THORNDIKE TWP | E | 211 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| FISH P | 3324 | LINCOLN PLT | D | 20 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| FISH P | 4054 | MOXIE GORE | D | 15 | S |  |  |  |  |  |  | 2 | INAC | DEV | 5 |
| FISH P | 6910 | HOBBSTOWN TWP | E | 275 |  |  | S |  |  | S |  | 2 | AC | UNDEV |  |
| FISH P (BIG) | 2660 | HOLEB TWP | E | 64 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| FISH P (LITTLE) | 2512 | ALDER BROOK TWP | E | 35 | S |  |  |  |  |  | S | 2 | INAC | UNDEV |  |
| FISH P (LITTLE) | 2666 | HOLEB TWP | E | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| FISH RIVER L | 0009 | T13 R08 WELS | G | 2642 | S | S | O- | S |  | S |  | 1A | AC | UNDEV | 3 |
| FISHER P #2 | 2560 | T04 R05 NBKP | E | 10 |  |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| FISHER P (BIG) | 2940 | T02 R12 WELS | E | 60 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| FISHER P (LITTLE) | 2942 | T02 R12 WELS | E | 35 |  |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| FLAGSTAFF L | 0038 | DEAD RIVER TWP | D | 20300 | O | O* | S | S |  |  |  | 1A | AC | UNDEV | 2 |
| FLATIRON P | 2006 | CEDAR LAKE TWP | F | 284 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| FLATIRON P | 2372 | DAVIS TWP | D | 30 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| FLETCHER P # 1 | 9736 | BRASSUA TWP | E | 12 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| FLETCHER P # 2 | 9734 | BRASSUA TWP | E | 20 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| FLINN P | 3036 | T01 R05 WELS | F | 269 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| FOGG P | 0426 | BOWDOIN COL GR WEST | E | 23 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| FOGG P | 2534 | LONG POND TWP | E | 54 |  |  | S | O |  |  |  | 1B | AC | UNDEV |  |
| FOLEY P | 2488 | COMSTOCK TWP | E | 124 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| FOLEY P (LITTLE) | 2492 | COMSTOCK TWP | E | 35 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| FOSS P | 0388 | KINGSBURY PLT | E | 110 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| FOURTH L | 1130 | T37 MD BPP | C | 32 | m | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| FOURTH L | 2820 | T07 R11 WELS | E | 64 | S |  | S | S |  |  |  | 2 | AC | UNDEV |  |
| FOWLER P | 0686 | T03 R11 WELS | E | 19 | S |  | O | S |  |  |  | 1B | INAC | UNDEV | 1/6 |
| FOX P | 1472 | T17 R12 WELS | G | 23 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| FOX P | 4438 | T10 SD | C | 77 | S | O | O | S |  |  |  | 1A | AC | UNDEV |  |
| FOX P | 7734 | LOWER CUPSUPTIC TWP | D | 20 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| FROST P | 0680 | T03 R11 WELS | E | 198 | S |  |  |  |  |  | O | 1B | AC | DEV |  |
| FROST P | 2455 | T04 R05 NBKP | E | 170 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| FROST P (LITTLE) | 0668 | T03 R12 WELS | E | 35 | S | S |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| FROST P FLOWAGE | 2562 | T04 R05 NBKP | E | 70 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| FRYPAN P | 4074 | SQUARETOWN TWP | D | 30 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| FURLONG P | 3488 | ALBANY TWP | A | 17 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| GALILEE P | 1526 | T15 R09 WELS | G | 9 | S |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| GAMMON P | 0024 | FREEMAN TWP | D | 97 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| GARDNER L | 1358 | MARION TWP | C | 3886 | O | O |  |  |  | S | O | 1A | AC | UNDEV |  |
| GARDNER L | 1528 | T15 R09 WELS | G | 288 | O | O | O |  |  |  |  | 1A | INAC | UNDEV | 1/6 |
| GARDNER P | 2192 | T05 R07 WELS | F | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| GASSABIAS L | 4782 | T41 MD | F | 896 | S |  | S |  |  | S |  | 2 | AC | UNDEV |  |
| GAUNTLET P | 0472 | TB R10 WELS | F | 11 | S |  | O |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| GERARD P | 1868 | CASWELL | G | 19 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| GETCHELL L (LOWER) | 1122 | T43 MD BPP | C | 58 | m |  |  |  | + | S |  | 2 | AC | UNDEV |  |
| GETCHELL L (UPPER) | 1120 | T43 MD BPP | C | 51 | m |  |  |  |  | S- |  | 2 | AC | UNDEV |  |
| GILBERT P | 1556 | ST JOHN PLT | G | 12 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| GILMAN P | 0004 | LEXINGTON TWP | D | 242 |  | O | S |  |  |  |  | 1B | AC | UNDEV |  |
| GILMAN P | 3638 | MORO PLT | G | 19 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| GLAZIER L | 9789 | T18 R10 WELS | G | 1120 | S |  |  |  | S |  |  | 2 | AC | UNDEV | 3 |
| GLAZIER P | 1898 | T11 R12 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| GOOSE P | 1218 | T24 MD BPP | C | 16 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| GORDON P | 0146 | UPPER ENCHANTED TWP | E | 28 | S |  |  |  |  |  |  | 3 | AC | UNDEV | 6 |
| GORDON P (LITTLE) | 0134 | LOWER ENCHANTED TWP | D | 15 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| GOULD P | 0620 | RAINBOW TWP | E | 12 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| GRACE P | 0152 | UPPER ENCHANTED TWP | E | 150 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| GRAHAM L | 4350 | FLETCHER’S LANDING TWP | C | 7865 | S | O |  |  |  | O |  | 1A | AC | DEV | 3 |
| GRAND FALLS FLOWAGE | 7437 | FOWLER TWP | C | 6691 | O | O |  |  |  |  |  | 1A | AC | UNDEV | 4 |
| GRAND L (EAST) | 1070 | FOREST CITY TWP | F | 16070 | O | O |  |  |  | S+ |  | 1A | AC | UNDEV |  |
| GRAND L (WEST) | 1150 | T06 ND BPP | C | 14340 | O | O | O | O |  | O |  | 1A | AC | DEV | 3 |
| GRAND LAKE SEBOIS | 3011 | T08 R07 WELS | G | 2483 | S | S | S |  |  | O |  | 1B | AC | UNDEV |  |
| GRANTS P | 3348 | MASSACHUSETTS GORE | D | 20 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| GRASS P | 0104 | PIERCE POND TWP | D | 70 | O | S |  |  |  |  |  | 1B | AC | UNDEV |  |
| GRASS P | 2778 | T09 R12 WELS | G | 50 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| GRASS P | 3646 | MORO PLT | G | 55 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| GRASSY P | 0660 | T02 R12 WELS | E | 54 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| GRASSY P | 1252 | T19 MD BPP | C | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| GRAY LEDGE DEADWATER | 9750 | CEDAR LAKE TWP | F | 70 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| GREAT WORKS P | 1386 | EDMUNDS TWP | C | 50 | S | O |  |  |  |  |  | 1B | INAC | UNDEV | 1 |
| GREELY P | 2380 | DALLAS PLT | D | 42 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| GREELY P (LITTLE) | 2382 | DALLAS PLT | D | 15 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| GREELY P (THIRD) | 2388 | DALLAS PLT | D | 14 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| GREEN L #1 | 4518 | T35 MD | C | 83 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| GREEN L #2 | 4790 | T35 MD | C | 64 |  |  | S |  |  |  |  | 2 | AC | DEV |  |
| GREEN MTN P | 3666 | T06 R06 WELS | F | 10 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| GREEN P | 2256 | T03 R01 NBPP | F | 110 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| GREEN P | 3648 | MORO PLT | G | 16 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| GREENBUSH P | 2346 | JIM POND TWP | D | 24 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| GREENLAW P | 1570 | T12 R07 WELS | G | 109 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| GREENWOOD P | 0464 | TB R11 WELS | F | 19 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| GREENWOOD P (BIG) | 0884 | ELLIOTTSVILLE TWP | E | 211 | O |  | O |  |  |  |  | 1A | AC | UNDEV | 4 |
| GREENWOOD P (LITTLE) | 0886 | ELLIOTTSVILLE TWP | E | 61 | S |  | O |  |  |  |  | 1B | AC | UNDEV |  |
| GRENIER P (BIG) | 2456 | T04 R05 NBKP | E | 110 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| GREY P | 1894 | T12 R13 WELS | G | 28 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| GROVER L (LITTLE) | 7325 | DAY BLOCK TWP | C | 17 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| GROVER P | 1244 | DAY BLOCK TWP | C | 68 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| GULL P | 3532 | DALLAS PLT | D | 281 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| HADLEY L #1 | 1224 | T24 MD BPP | C | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HADLEY L #2 | 1226 | T24 MD BPP | C | 36 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HAFEY P | 1498 | T18 R11 WELS | G | 23 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| HALE P | 2062 | T02 R10 WELS | F | 168 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| HALE P | 2508 | ALDER BROOK TWP | E | 40 | m+ |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| HALE P | 3652 | MORO PLT | G | 38 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| HALEY P | 3534 | DALLAS PLT | D | 170 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| HALL P | 2566 | PRENTISS TWP T04 R04 NBKP | E | 19 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| HALL P | 5092 | T05 R07 BKP WKR | E | 42 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| HAMMOND P | 7431 | MARION TWP | C | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HANCOCK P | 0082 | LEXINGTON TWP | D | 320 | S |  |  |  |  |  | S | 2 | AC | DEV |  |
| HARDY P | 0958 | LAKE VIEW PLT | F | 125 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HARRINGTON L | 0700 | T03 R11 WELS | E | 1332 | O | S | S | S |  | S | O | 1A | AC | UNDEV |  |
| HARRINGTON P | 0702 | T03 R11 WELS | E | 40 | m |  | O |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| HARROW L | 1934 | T10 R11 WELS | G | 467 | S | S | S |  |  | S |  | 1B | AC | UNDEV |  |
| HARROW L (LITTLE) | 1936 | T10 R11 WELS | G | 40 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| HATHAN BOG (UPPER) | 8837 | COBURN GORE | D | 55 | m | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| HATHORN P | 4242 | T04 R08 WELS | F | 15 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| HATHORN P (LITTLE) | 2298 | T04 R08 WELS | F | 8 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| HAY L | 2178 | T06 R08 WELS | F | 588 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| HAY P | 2824 | T07 R11 WELS | E | 19 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| HAY P | 4252 | T06 R08 WELS | F | 134 | S | S | S |  |  |  |  | 2 | AC | UNDEV |  |
| HAYDEN P | 0264 | MAYFIELD TWP | D | 10 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| HAYMOCK L | 2814 | T07 R11 WELS | E | 704 | S |  | S | S |  | S |  | 1B | AC | UNDEV |  |
| HAYWIRE P | 9769 | T01 R05 WELS | F | 25 |  |  |  |  |  |  |  | 3 | INAC | DEV |  |
| HEDGEHOG P | 0284 | BLANCHARD TWP | E | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HEDGEHOG P | 0556 | T01 R11 WELS | E | 5 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| HEDGEHOG P | 0790 | BOWDOIN COL GR WEST | E | 40 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| HELEN P | 0094 | PIERCE POND TWP | D | 15 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| HENDERSON P | 0532 | TA R11 WELS | E | 195 | S |  | S | S |  |  |  | 2 | INAC | UNDEV |  |
| HEWES BROOK P | 1606 | T14 R07 WELS | G | 24 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HIGH P | 0092 | PIERCE POND TWP | D | 7 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| HILLS P | 3686 | PERKINS TWP T07 R03 NBPP | D | 22 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| HILTON P # 1 | 0304 | KINGSBURY PLT | E | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HOBART BOG | 7451 | EDMUNDS TWP | C | 30 | S | O |  |  |  |  |  | 1B | INAC | UNDEV | 1 |
| HOBART L | 1388 | EDMUNDS TWP | C | 90 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HOBART P | 2166 | T06 R07 WELS | F | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HOLBROOK P | 0632 | RAINBOW TWP | E | 224 | S |  | S | O |  |  |  | 1B | INAC | UNDEV | 6 |
| HOLBROOK P (LITTLE) | 9708 | T03 R11 WELS | E | 46 |  |  | S | S |  |  |  | 2 | INAC | UNDEV |  |
| HOLEB P | 2652 | HOLEB TWP | E | 1055 | S |  | O | O | O |  | P | 1A | AC | DEV | 4 |
| HORSE L | 1136 | T37 MD BPP | C | 26 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HORSEBACK P | 2164 | T07 R07 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HORSERACE PONDS | 0626 | RAINBOW TWP | E | 50 | O |  | O | S |  |  | O | 1A | INAC | UNDEV | 1/6 |
| HORSESHOE L | 4706 | LAKEVILLE | F | 400 | S |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| HORSESHOE L | 4788 | T35 MD | C | 202 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HORSESHOE P | 0102 | PIERCE POND TWP | D | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HORSESHOE P | 0412 | BOWDOIN COL GR WEST | E | 160 | O |  | S+ | S- |  |  | S- | 1B | AC | DEV |  |
| HORSESHOE P | 1926 | T11 R10 WELS | G | 23 | S |  | O | S |  |  |  | 1B | AC | UNDEV |  |
| HORSESHOE P | 2540 | PARLIN POND TWP | E | 50 | m |  | S+ | S+ |  |  |  | 2 | INAC | UNDEV |  |
| HORSESHOE P | 2686 | ATTEAN TWP | E | 50 | m+ |  |  | + |  |  |  | 3 | INAC | UNDEV | 6 |
| HORSESHOE P | 3336 | COBURN GORE | D | 37 |  | S |  |  |  |  |  | 2 | AC | UNDEV | 3 |
| HORSESHOE P | 4082 | CHASE STREAM TWP | D | 27 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| HORSESHOE P | 4102 | EAST MOXIE TWP | D | 25 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HORSESHOE P | 5150 | T03 R05 BKP WKR | D | 29 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| HORSESHOE P | 9277 | T16 R09 WELS | G | 15 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| HORSESHOE P | 9823 | T18 MD BPP | C | 12 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HOT BROOK L (LOWER) | 1072 | T08 R04 NBPP | F | 713 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HOT BROOK L (UPPER) | 1076 | T08 R04 NBPP | F | 912 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HOT P | 2168 | T06 R07 WELS | F | 70 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HOULTON P | 1460 | T13 R16 WELS | G | 19 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HOUND BROOK L | 1326 | DYER TWP | C | 140 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HOUSTON P | 0916 | T07 R09 NWP | E | 694 | S |  | S |  |  |  | S- | 2 | AC | UNDEV |  |
| HOUSTON P (LITTLE) | 0920 | KATAHDIN IRN WKS TWP | F | 27 | O |  |  |  |  |  | S | 1B | INAC | UNDEV | 6 |
| HUDSON P (LITTLE) | 2742 | T10 R10 WELS | G | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| HUDSON P (LOWER) | 1954 | T10 R10 WELS | G | 108 | S |  | O | S |  |  |  | 1B | AC | UNDEV |  |
| HUDSON P (UPPER) | 1928 | T11 R10 WELS | G | 32 | O |  | O |  |  |  |  | 1A | INAC | UNDEV | 1 |
| HUMPBACK BOG | 4488 | T28 MD | C | 32 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| HUNNEWELL L | 1554 | ST JOHN PLT | G | 64 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| HUNTLEY P | 3640 | MORO PLT | G | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HURD P | 2064 | T02 R10 WELS | F | 640 | S |  | O | S |  |  |  | 1B | AC | DEV |  |
| HURD P (BIG) | 4014 | T06 R15 WELS | E | 250 | S |  | O |  |  |  | S | 1B | AC | UNDEV |  |
| HURD P (LITTLE) | 0596 | T02 R10 WELS | F | 60 | S |  | O | S |  |  | S | 1B | INAC | UNDEV | 1/6 |
| HURD P (LITTLE) | 4016 | T06 R15 WELS | E | 180 |  | S | S |  |  |  |  | 2 | INAC | UNDEV |  |
| HURRICANE P | 5046 | KIBBY TWP | D | 20 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| HURRICANE P | 5166 | T05 R20 WELS | E | 54 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| HUSSEY P | 0292 | BLANCHARD TWP | E | 15 |  |  |  |  |  |  |  | 3 | INAC | DEV |  |
| HUSTON BROOK P | 7304 | DEAD RIVER TWP | D | 7 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| HUTCH P | 2498 | HAMMOND TWP | E | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HUTCHINSON P | 0492 | TA R11 WELS | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| HUTCHINSON P | 3494 | ALBANY TWP | A | 96 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| IMLOS P | 2714 | T06 R11 WELS | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| INDIAN & DAM PONDS | 0814 | T07 R09 NWP | E | 299 | S |  | S |  |  | S |  | 2 | AC | UNDEV |  |
| INDIAN L | 1242 | T19 ED BPP | C | 13 |  |  |  |  |  | S- |  | 2 | INAC | UNDEV |  |
| INDIAN P | 0002 | LEXINGTON TWP | D | 53 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| INDIAN P | 0782 | BOWDOIN COL GR WEST | E | 70 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| INDIAN P | 2866 | T07 R12 WELS | E | 1222 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| INDIAN P | 4090 | SAPLING TWP | E | 3746 | S | O |  |  |  | S |  | 1B | AC | DEV | 3 |
| INDIAN P (BIG) | 0324 | MOOSEHEAD JUNCTION TWP | E | 280 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| INDIAN P (LITTLE) | 2808 | T07 R12 WELS | E | 117 |  |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| INDIAN P (LITTLE) | 4070 | SQUARETOWN TWP | D | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| IRA BOG | 0332 | MOOSEHEAD JUNCTION TWP | E | 28 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| IRELAND P | 4168 | T07 R08 WELS | F | 30 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| IRON P | 5106 | HAYNESTOWN TWP | E | 32 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| IRONBOUND P | 2510 | ALDER BROOK TWP | E | 40 | O |  | O | O |  |  | O | 1A | AC | UNDEV | 2 |
| ISLAND (CHASE) P | 1942 | T10 R10 WELS | G | 214 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| ISLAND P | 1516 | T15 R09 WELS | G | 32 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ISLAND P | 1586 | T14 R08 WELS | G | 27 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ISLAND P | 4094 | CHASE STREAM TWP | D | 24 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| ISLAND P (BIG) | 3352 | SEVEN PONDS TWP | D | 350 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| ISLAND P (LITTLE) | 3350 | SEVEN PONDS TWP | D | 50 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ISLAND P (UPPER) | 2740 | T10 R10 WELS | G | 45 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ISTHMUS P | 2000 | T08 R08 WELS | G | 23 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| JACK P | 4140 | T07 R10 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| JACKSON BROOK L | 1334 | FOREST TWP | F | 480 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| JACKSON P | 0182 | CONCORD TWP | D | 32 |  |  | O | S |  |  |  | 1B | AC | UNDEV |  |
| JACKSON P #1 | 0684 | T03 R11 WELS | E | 23 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| JACKSON P #2 | 0704 | T03 R11 WELS | E | 12 | S |  | O | O |  |  |  | 1A | AC | UNDEV | 2 |
| JERRY P | 2122 | MILLINOCKET | F | 66 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| JERRY P | 2190 | T05 R07 WELS | F | 272 | S |  | O | S |  |  |  | 1B | INAC | UNDEV | 1 |
| JEWETT P | 0050 | PLEASANT RIDGE PLT | D | 32 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| JEWETT P | 0460 | SPENCER BAY TWP | E | 13 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| JIM P | 5054 | JIM POND TWP | D | 320 | O | O | O | S+ |  |  |  | 1A | AC | UNDEV | 2 |
| JIM P (LITTLE) | 5090 | JIM POND TWP | D | 64 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| JIMMIES P | 8259 | T34 MD | C | 26 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| JO-MARY L (LOWER) | 0984 | T01 R10 WELS | F | 1910 | S |  | O |  |  | S | S | 1B | INAC | UNDEV | 1 |
| JO-MARY L (MIDDLE) | 0986 | T04 INDIAN PURCHASE TWP | F | 1152 | S |  | O | S |  | S | S | 1A | AC | DEV | 3 |
| JO-MARY L (UPPER) | 0243 | TA R10 WELS | F | 1873 | O |  | O | S |  |  | S | 1A | AC | UNDEV | 2 |
| JO-MARY P | 0476 | TB R10 WELS | F | 38 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| JO-MARY S DWTR (UP) | 9630 | T04 INDIAN PURCHASE TWP | F | 30 |  |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| JOE POKUM P | 5126 | KING & BARTLETT TWP | D | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| JOHNS P | 0144 | LOWER ENCHANTED TWP | D | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| JOHNS P | 3956 | DAVIS TWP | D | 267 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| JOHNSON P | 2986 | T08 R14 WELS | E | 197 | S |  | O | S |  |  | O | 1A | AC | UNDEV |  |
| JOHNSTON P | 0534 | TA R10 WELS | F | 59 | S |  | O |  |  |  |  | 1B | AC | UNDEV |  |
| JONES P | 0172 | WYMAN TWP | D | 36 |  | O |  |  |  |  |  | 1B | INAC | UNDEV | 1 |
| JONES P | 1500 | BIG TWENTY TWP | G | 77 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| JONES P | 2486 | BALD MTN TWP T4R3 | E | 130 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| JONES P | 3002 | T07 R08 WELS | F | 32 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| JUNIOR L | 4708 | PUKAKON TWP | F | 3866 | S |  | S | S |  | S |  | 1B | AC | UNDEV |  |
| JUNIPER KNEE P | 0878 | ELLIOTTSVILLE TWP | E | 32 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| KAMANKEAG P | 3954 | DAVIS TWP | D | 40 | O |  | S |  |  |  |  | 1B | AC | UNDEV |  |
| KATAHDIN L | 2016 | T03 R08 WELS | F | 717 | S |  | O | O |  | S | S | 1A | INAC | UNDEV | 1 |
| KEG L | 4700 | LAKEVILLE | F | 378 | S |  | S | S |  |  |  | 2 | INAC | UNDEV |  |
| KELLY P | 0654 | T02 R12 WELS | E | 60 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| KELLY P (LITTLE) | 0658 | T02 R12 WELS | E | 21 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| KENNEBAGO L (BIG) | 2374 | DAVIS TWP | D | 1700 | O | O | O | O |  | S- | O | 1A | AC | DEV | 4 |
| KENNEBAGO L (LITTLE) | 3958 | STETSONTOWN TWP | D | 190 | O |  |  |  |  | + |  | 1B | AC | DEV |  |
| KIDNEY P | 2926 | E MIDDLESEX CANAL GR | E | 45 | m | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| KILGORE P | 0108 | PIERCE POND TWP | D | 96 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| KILGORE P (UPPER) | 0106 | BOWTOWN TWP | D | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| KIMBALL P | 4245 | T05 R08 WELS | F | 64 | S |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| KING & BARTLETT L | 5136 | KING & BARTLETT TWP | D | 538 | O |  |  |  |  |  | S | 1B | AC | UNDEV |  |
| KING L (LITTLE) | 5138 | KING & BARTLETT TWP | D | 90 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| KING P | 0111 | BOWTOWN TWP | D | 16 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| KINGSBURY P | 0262 | MAYFIELD TWP | D | 390 | S |  |  |  |  |  | S | 2 | AC | DEV | 5 |
| KINGSLEY BOG | 0276 | MAYFIELD TWP | D | 10 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| KINGSLEY FLOWAGE | 7148 | MAYFIELD TWP | D | 14 |  | S |  |  |  |  | S | 2 | INAC | UNDEV |  |
| KNEELAND P | 3266 | ALBANY TWP | A | 16 | m |  | S |  |  |  |  | 2 | AC | DEV | 5 |
| KNIGHTS P | 0377 | SQUARETOWN TWP | D | 128 |  |  |  |  | S |  |  | 2 | INAC | UNDEV |  |
| KNOX L | 1142 | T36 MD BPP | C | 51 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| KYLE P | 2810 | T07 R12 WELS | E | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| L POND | 5062 | SEVEN PONDS TWP | D | 95 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| LA POMKEAG L (LOWER) | 3010 | T08 R07 WELS | G | 91 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LA POMKEAG L (UPPER) | 3012 | T08 R07 WELS | G | 217 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LAMBERT L | 1332 | T11 R03 NBPP | F | 605 | S |  | S | S | S |  |  | 1B | AC | DEV |  |
| LANE BROOK P | 3664 | T06 R06 WELS | F | 33 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| LANE P | 2490 | COMSTOCK TWP | E | 24 | S |  |  |  |  |  | O | 1B | INAC | UNDEV | 1/6 |
| LANE P (LITTLE) | 2502 | HAMMOND TWP | E | 18 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LANG P | 2542 | PARLIN POND TWP | E | 30 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| LANG P (LITTLE) | 2543 | PARLIN POND TWP | E | 13 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| LARD P | 2442 | ELM STREAM TWP | E | 14 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LAZY TOM P | 0458 | T01 R13 WELS | E | 17 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| LEAD MTN P (LO & MD) | 4484 | T28 MD | C | 486 | m |  | S |  |  |  |  | 2 | AC | DEV |  |
| LEAD MTN P (UPPER) | 4482 | T28 MD | C | 1021 | S |  | S | S |  |  |  | 2 | AC | DEV |  |
| LEADBETTER P | 2764 | T09 R11 WELS | G | 98 | S | S | S- |  |  |  |  | 2 | AC | UNDEV |  |
| LEADBETTER P | 2880 | T07 R12 WELS | E | 135 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LEADBETTER P (LT) | 2700 | T07 R11 WELS | E | 147 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LEAVITT P | 0546 | T01 R11 WELS | E | 50 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LEDGE P | 3554 | SANDY RIVER PLT | D | 6 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| LEITH P | 4124 | BRASSUA TWP | E | 18 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| LEONARD P | 1988 | T10 R09 WELS | G | 38 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| LEWIS P | 2862 | SOPER MOUNTAIN TWP | G | 30 |  |  |  |  |  |  | P | 3 | INAC | UNDEV |  |
| LILY (MERRILL) P | 0180 | CONCORD TWP | D | 25 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| LILY L | 1164 | T30 MD BPP | C | 32 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LILY L | 1396 | TRESCOTT TWP | C | 31 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LILY P | 0178 | CONCORD TWP | D | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LINCOLN P | 0988 | T04 INDIAN PURCHASE TWP | F | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LINCOLN P | 3964 | PARKERTOWN TWP | D | 340 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| LINE P | 2806 | T07 R11 WELS | E | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LINE P | 5162 | T05 R20 WELS | E | 7 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| LINSCOTT P | 1452 | T13 R16 WELS | G | 75 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LITTLE BOG | 2404 | T06 R17 WELS | E | 128 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LITTLE L | 7513 | BARING PLT | C | 275 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LITTLE P | 0302 | KINGSBURY PLT | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LLOYD P | 0462 | BOWDOIN COL GR EAST | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LOBSTER L | 2948 | LOBSTER TWP | E | 3475 | O | O | O | O | O | S | O | 1A | AC | UNDEV | 2 |
| LOBSTER L (LITTLE) | 2946 | LOBSTER TWP | E | 230 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LOGAN P # 1 | 2080 | T02 R09 WELS | F | 29 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOGAN P # 2 | 2082 | T02 R09 WELS | F | 20 |  |  | O | S |  |  |  | 1B | INAC | UNDEV | 1 |
| LOMBARD L | 4690 | LAKEVILLE | F | 225 | S |  | O |  |  |  |  | 1B | AC | UNDEV |  |
| LONE JACK P | 2556 | JOHNSON MOUNTAIN TWP | E | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LONG (LITTLE LONG) P | 5794 | CHASE STREAM TWP | D | 17 | S |  | + |  |  |  |  | 2 | AC | UNDEV |  |
| LONG (MARTIN) P | 4108 | THE FORKS PLT | D | 26 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| LONG BOG | 0450 | SHAWTOWN TWP | E | 15 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LONG BOG | 2668 | HOLEB TWP | E | 19 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| LONG L | 1264 | T19 ED BPP | C | 698 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LONG L | 1370 | MARION TWP | C | 130 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LONG L | 1682 | VAN BUREN COVE TWP | G | 6000 | S |  |  |  |  | S |  | 2 | AC | DEV |  |
| LONG L | 1892 | T12 R13 WELS | G | 1203 | O | O* |  |  |  | S | S | 1A | AC | UNDEV | 2 |
| LONG P | 0506 | TA R11 WELS | E | 371 | O |  |  |  |  |  | S | 1B | AC | UNDEV |  |
| LONG P | 0800 | T07 R09 NWP | E | 643 | S |  | S |  |  | S- |  | 2 | AC | UNDEV |  |
| LONG P | 1200 | T18 MD BPP | C | 15 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| LONG P | 1596 | T13 R08 WELS | G | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LONG P | 1922 | T11 R10 WELS | G | 128 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| LONG P | 2536 | LONG POND TWP | E | 3053 | S | S | O | S |  | S |  | 1A | AC | DEV | 3 |
| LONG P | 2646 | FORSYTH TWP | E | 98 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LONG P | 2690 | ATTEAN TWP | E | 37 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| LONG P | 3116 | LYNCHTOWN TWP | D | 8 |  |  |  |  |  | S- |  | 2 | AC | UNDEV |  |
| LONG P | 3320 | MAGALLOWAY TWP | D | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LONG P | 3356 | SEVEN PONDS TWP | D | 35 | S |  |  |  |  |  |  | 2 | AC | UNDEV | 3 |
| LONG P | 3582 | TOWNSHIP E | D | 254 | S |  |  |  |  |  |  | 2 | INAC | DEV |  |
| LONG P | 4118 | TAUNTON & RAYNHAM | E | 173 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| LONG P | 4430 | T10 SD | C | 205 | S |  | S | S |  |  |  | 2 | AC | UNDEV |  |
| LONG P | 5124 | KING & BARTLETT TWP | D | 60 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| LONG P | 9861 | DOLE BROOK TWP | E | 845 | S |  | S | S |  |  |  | 2 | AC | UNDEV |  |
| LONG P (LITTLE) | 4424 | T10 SD | C | 55 | S |  | O | S |  |  |  | 1B | INAC | UNDEV | 1/6 |
| LONGLEY P | 2886 | T06 R13 WELS | E | 749 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| LONGLEY P (LITTLE) | 2888 | T06 R13 WELS | E | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOON L | 2384 | DALLAS PLT | D | 168 | S |  |  |  |  | S- |  | 2 | AC | DEV | 5 |
| LOON L | 4024 | T06 R15 WELS | E | 1140 | S | O | S | S |  |  | O | 1A | AC | UNDEV |  |
| LOON P | 0453 | T40 MD | F | 13 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LOON P | 0554 | T01 R11 WELS | E | 5 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| LOON P | 2688 | ATTEAN TWP | E | 37 | O |  | + | + | + |  |  | 1B | INAC | UNDEV | 1/6 |
| LOST P | 0208 | PLEASANT RIDGE PLT | D | 18 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| LOST P | 0820 | T07 R09 NWP | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 1658 | T15 R06 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 1690 | ISLAND FALLS | F | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 1924 | T11 R10 WELS | G | 58 | + |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 1984 | T10 R08 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 2420 | RUSSELL POND TWP | E | 45 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LOST P | 2526 | THORNDIKE TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 2694 | ATTEAN TWP | E | 5 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| LOST P | 2878 | T07 R13 WELS | E | 47 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 3030 | T07 R07 WELS | F | 38 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LOST P | 3042 | UPPER MOLUNKUS TWP | F | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LOST P | 5146 | T03 R05 BKP WKR | D | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LOVE L | 1238 | T19 ED BPP | C | 672 | S |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| LOVEJOY P | 4506 | T34 MD | C | 40 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| LOWELL L | 4680 | CARROLL PLT | F | 115 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LOWER P | 8255 | T35 MD | C | 16 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| LUCIA P | 0810 | T07 R09 NWP | E | 46 |  |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| LUCIFEE P | 9755 | T14 R08 WELS | G | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| LUCKY P | 0402 | SPENCER BAY TWP | E | 93 |  | S |  |  |  |  | S | 2 | AC | UNDEV |  |
| LUNKSOOS L | 2206 | T04 R07 WELS | F | 288 | S |  | S | S |  |  |  | 2 | INAC | UNDEV |  |
| LUTHER P | 2528 | THORNDIKE TWP | E | 154 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LYFORD P (BIG) | 0438 | SHAWTOWN TWP | E | 152 | O |  |  |  |  | O |  | 1A | AC | DEV | 4 |
| LYFORD P(1ST LITTLE) | 0430 | BOWDOIN COL GR EAST | E | 21 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| LYFORD P(2ND LITTLE) | 0428 | BOWDOIN COL GR EAST | E | 18 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MACHIAS L (BIG) | 1960 | T12 R08 WELS | G | 692 | S | S |  |  |  | S |  | 2 | AC | UNDEV | 3 |
| MACHIAS L (FIFTH) | 1144 | T36 MD BPP | C | 1069 | m |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| MACHIAS L (FIRST) | 1140 | T37 MD BPP | C | 109 | S |  |  |  |  | S |  | 2 | AC | DEV |  |
| MACHIAS L (FOURTH) | 1148 | SAKOM TWP | C | 1539 | O | S | + |  |  | S |  | 1B | AC | UNDEV |  |
| MACHIAS L (LITTLE) | 1578 | NASHVILLE PLT | G | 275 | S | S |  |  |  |  |  | 2 | AC | UNDEV | 3 |
| MACHIAS L (SECOND) | 1138 | T37 MD BPP | C | 192 | S |  |  |  |  | S |  | 2 | AC | DEV |  |
| MACHIAS L (THIRD) | 1124 | T42 MD BPP | C | 2778 | O | O | + | + | + | S | + | 1A | AC | UNDEV | 2 |
| MACWAHOC L (LOWER) | 3058 | T02 R04 WELS | F | 188 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MACWAHOC L (UPPER) | 3052 | T03 R04 WELS | F | 269 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MADAGASCAL P (BIG) | 2254 | T03 R01 NBPP | F | 750 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MADAGASCAL P(LITTLE) | 2258 | T03 R01 NBPP | F | 40 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MADAWASKA L | 1802 | MADAWASKA LAKE TWP | G | 1526 | S | S |  |  | O | S |  | 1B | AC | DEV | 5 |
| MARBLE P | 0280 | BLANCHARD TWP | E | 15 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MARBLE P | 2186 | T05 R08 WELS | F | 75 | S |  | S | S | O |  | O | 1A | INAC | UNDEV | 1 |
| MARCIAL L | 1636 | WALLAGRASS | G | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MARTIN L | 1858 | CASWELL | G | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MARY PETUCHE P | 2474 | PRENTISS TWP T04 R04 NBKP | E | 10 | S |  |  |  |  |  | O | 1B | INAC | UNDEV | 1/6 |
| MASSACHUSETTS BOG | 8597 | MASSACHUSETTS GORE | D | 30 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| MATAGAMON L | 4260 | T06 R08 WELS | F | 4165 | S | O | S | S | O | O |  | 1A | AC | UNDEV |  |
| MATHERSON P | 3072 | T09 R05 WELS | G | 45 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MATHEWS P | 2836 | T08 R10 WELS | G | 19 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1 |
| MATTAMISCONTIS L | 2140 | CEDAR LAKE TWP | F | 1025 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| MATTAMISCONTIS L(LT) | 2138 | CEDAR LAKE TWP | F | 275 | S |  |  |  |  |  |  | 2 | AC | UNDEV | 3 |
| MATTASEUNK L | 3040 | MOLUNKUS TWP | F | 576 | S |  |  |  |  |  |  | 2 | AC | DEV | 3 |
| MATTAWAMKEAG L | 1686 | T04 R03 WELS | F | 3330 | S | O | S | O |  | S | S | 1A | AC | UNDEV |  |
| MAY P | 2826 | T07 R11 WELS | E | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MAYFIELD P | 0260 | MAYFIELD TWP | D | 140 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| MCCLUSKY L | 1660 | T14 R05 WELS | G | 49 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MCDOUGAL P | 4022 | T06 R15 WELS | E | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MCGILVRY P | 3082 | T08 R06 WELS | G | 25 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MCGOOSELEY P | 4032 | T04 R15 WELS | E | 38 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| MCGOWAN P | 1962 | T11 R08 WELS | G | 160 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MCKEEN L | 1476 | T14 R10 WELS | G | 147 | S | + |  |  |  |  |  | 2 | AC | UNDEV |  |
| MCKENNA P | 0688 | T03 R11 WELS | E | 53 | m |  | O | S |  |  |  | 1B | INAC | UNDEV | 1/6 |
| MCKENNEY P | 0154 | UPPER ENCHANTED TWP | E | 9 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| MCKENNEY P | 2650 | HOLEB TWP | E | 38 | S |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| MCLELLAN P | 0364 | ELLIOTTSVILLE TWP | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MCNALLY P (UPPER) | 1930 | T11 R10 WELS | G | 83 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MCPHERSON P | 1992 | T10 R10 WELS | G | 77 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MEDDYBEMPS L | 0177 | BARING PLT | C | 6765 | O |  |  |  |  | S |  | 1B | AC | UNDEV |  |
| MEDUNKEUNK L | 2132 | T02 R09 NWP | F | 67 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| MESSER P | 4244 | T05 R08 WELS | F | 27 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| MESSER P (LITTLE) | 5806 | T05 R08 WELS | F | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| METALLAK P | 3326 | TOWNSHIP C | D | 75 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MIDDAY P | 0890 | ELLIOTTSVILLE TWP | E | 12 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MIDDLE DEADWATER | 1099 | KOSSUTH TWP | F | 50 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MIDNIGHT P | 2708 | T06 R12 WELS | E | 64 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MIDWAY P | 3544 | SANDY RIVER PLT | D | 7 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| MILE P | 2982 | T08 R14 WELS | E | 66 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MILL (CLEAR) P | 0200 | PLEASANT RIDGE PLT | D | 23 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MILL PRIVILEGE L | 4704 | PUKAKON TWP | F | 109 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MILLIMAGASSETT L | 3004 | T07 R08 WELS | F | 1410 | S | O | + | + |  |  |  | 1B | INAC | UNDEV | 1 |
| MILLINOCKET L | 2020 | T01 R08 WELS | F | 8960 | S | O | S+ | S+ | + | S | S+ | 1A | AC | UNDEV |  |
| MILLINOCKET L | 4156 | T07 R09 WELS | F | 2701 | S | S+ | S | S | + | O | + | 1A | AC | UNDEV |  |
| MINISTER L (LITTLE) | 0592 | T02 R10 WELS | F | 4 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| MINISTER P (BIG) | 0590 | T02 R10 WELS | F | 15 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| MINK MARSH P | 1982 | T10 R10 WELS | G | 13 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| MINK P | 1478 | T14 R10 WELS | G | 21 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MISERY P | 5800 | MISERY TWP | E | 36 | S |  | S- | S+ |  |  | + | 2 | AC | UNDEV |  |
| MISERY P (UPPER) | 5802 | MISERY TWP | E | 18 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MITCHELL P | 9757 | T07 R09 WELS | F | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MOCCASIN P | 1590 | T14 R08 WELS | G | 32 | O |  |  |  |  |  |  | 1B | AC | UNDEV | 1 |
| MOLUNKUS L | 3038 | MOLUNKUS TWP | F | 1050 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| MONKEY P | 0526 | TA R11 WELS | E | 30 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| MONROE L | 1126 | T43 MD BPP | C | 45 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| MONROE P (EAST) | 9641 | T43 MD BPP | C | 10 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MONROE P (WEST) | 9811 | T43 MD BPP | C | 13 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MONTEGAIL P | 1196 | T19 MD BPP | C | 170 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MOORE P | 5096 | BRADSTREET TWP | E | 47 | S |  |  |  | O |  |  | 1B | AC | UNDEV |  |
| MOOSE BOG | 7688 | BOWMANTOWN TWP | D | 40 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| MOOSE P | 0118 | BOWTOWN TWP | D | 11 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| MOOSE P | 0902 | ELLIOTTSVILLE TWP | E | 14 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MOOSE P (BIG) | 0334 | MOOSEHEAD JUNCTION TWP | E | 91 | O |  |  |  |  |  | S | 1B | INAC | UNDEV | 1/6 |
| MOOSE P (LITTLE) | 0336 | MOOSEHEAD JUNCTION TWP | E | 25 | O |  |  |  |  |  | S | 1B | INAC | UNDEV | 1/6 |
| MOOSE P (LITTLE) | 4152 | T07 R10 WELS | F | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MOOSE P (UPPER) | 2828 | T07 R10 WELS | F | 35 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MOOSEHEAD L | 0390 | MOOSEHEAD JUNCTION TWP | E | 74890 | O | O | O | O | O | O | O | 1A | AC | DEV |  |
| MOOSEHEAD L #1 | MH01 | COVE POINT TWP | E | 6091 | O | O | O | S | O | O | O | 1A | AC | DEV |  |
| MOOSEHEAD L #2 | MH02 | BEAVER COVE | E | 12066 | O | O | O | S | O | O | O | 1A | AC | DEV |  |
| MOOSEHEAD L #3 | MH03 | LILY BAY TWP | E | 6072 | O | O | O | S | O | O | O | 1A | AC | DEV |  |
| MOOSEHEAD L #4 | MH04 | SANDBAR TRACT TWP | E | 12046 | O | O | S | S | O | O | O | 1A | AC | DEV |  |
| MOOSEHEAD L #5 | MH05 | SPENCER BAY TWP | E | 4710 | O | O | O | S | O | O | O | 1A | AC | UNDEV |  |
| MOOSEHEAD L #6 | MH06 | TOMHEGAN TWP | E | 9925 | O | O | O |  | O | O | O | 1A | AC | DEV |  |
| MOOSEHEAD L #7 | MH07 | DAYS ACADEMY GRANT | E | 8037 | O | O | O | S | O | O | O | 1A | AC | DEV |  |
| MOOSEHEAD L #8 | MH08 | NORTHEAST CARRY TWP | E | 15802 | O | O | O | O | O | O | O | 1A | AC | DEV |  |
| MOOSELEUK L | 1990 | T10 R09 WELS | G | 422 | S | O* | O- |  |  | O |  | 1A | AC | UNDEV | 2 |
| MOOSELOOKMEGUNTIC L* | 3302 | RICHARDSONTOWN TWP | D | 16300 | O | O | O | O |  | O |  | 1A | AC | DEV |  |
| MOOSELOOKMEGUNTIC L* | MLML | RICHARDSONTOWN TWP | D | 14101 | O | O | S | O |  | O |  | 1A | AC | DEV |  |
| MOPANG L | 1172 | DEVEREAUX TWP | C | 1487 | O |  | S | S |  |  |  | 1B | AC | UNDEV |  |
| MOPANG L (FIRST) | 7339 | T30 MD BPP | C | 27 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| MOPANG L (SECOND) | 1170 | DEVEREAUX TWP | C | 145 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MORANCY P | 4394 | T07 SD | C | 64 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MORRELL P | 4262 | T07 R08 WELS | F | 40 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MOSQUITO BROOK P | 1604 | T14 R07 WELS | G | 10 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MOSQUITO P | 4052 | THE FORKS PLT | D | 71 | S | S | O | S |  |  |  | 1B | AC | UNDEV |  |
| MOUNTAIN BROOK P | 0414 | BOWDOIN COL GR WEST | E | 21 | O |  | + |  |  |  |  | 1B | AC | UNDEV |  |
| MOUNTAIN CATCHER P | 4258 | T06 R08 WELS | F | 84 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| MOUNTAIN P | 0432 | BEAVER COVE | E | 56 | S |  |  |  |  |  | S | 2 | INAC | UNDEV | 6 |
| MOUNTAIN P | 1956 | T10 R10 WELS | G | 80 | S | S | S | + |  | + |  | 2 | AC | UNDEV |  |
| MOUNTAIN P | 2989 | T08 R15 WELS | E | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MOUNTAIN P | 3540 | RANGELEY PLT | D | 35 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| MOUNTAIN VIEW P | 0269 | BIG MOOSE TWP | E | 550 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MOUNTAIN VIEW P | 0488 | TA R11 WELS | E | 13 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| MOXIE BOG | 7106 | BALD MTN TWP T2R3 | D | 15 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MOXIE P | 3585 | TOWNSHIP D | D | 6 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| MOXIE P | 4050 | EAST MOXIE TWP | D | 2370 | S |  | O | S |  | S | P | 1B | AC | DEV |  |
| MOXIE P (LITTLE) | 0316 | EAST MOXIE TWP | D | 73 | S |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| MT CATCHER P(LITTLE) | 9922 | T06 R08 WELS | F | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD BROOK FLOWAGE | 9632 | T02 R08 WELS | F | 50 |  |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| MUD GAUNTLET DEADWTR | 0470 | TB R11 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD L | 1680 | SINCLAIR TWP | G | 972 | S |  |  |  |  | S |  | 2 | AC | DEV |  |
| MUD L | 1688 | T03 R03 WELS | F | 122 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD L | 1732 | T04 R03 WELS | F | 153 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD L | 1866 | CASWELL | G | 19 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD L | 7399 | FOREST CITY TWP | F | 291 |  |  |  |  |  |  | + | 3 | AC | DEV |  |
| MUD P | 0330 | MOOSEHEAD JUNCTION TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 0398 | BEAVER COVE | E | 249 |  | S |  |  |  |  | S | 2 | INAC | UNDEV |  |
| MUD P | 0542 | TA R10 WELS | F | 225 |  |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| MUD P | 1496 | T19 R11 WELS | G | 21 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 1532 | T15 R09 WELS | G | 63 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 1546 | T13 R08 WELS | G | 30 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| MUD P | 1600 | T13 R07 WELS | G | 19 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MUD P | 1874 | T12 R17 WELS | G | 49 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUD P | 2066 | T02 R10 WELS | F | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 2182 | T06 R08 WELS | F | 254 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUD P | 2340 | TOWNSHIP 6 N OF WELD | D | 6 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| MUD P | 2360 | TIM POND TWP | D | 30 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| MUD P | 2530 | THORNDIKE TWP | E | 50 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUD P | 2618 | BALD MTN TWP T4R3 | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 2648 | HOLEB TWP | E | 17 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUD P | 2696 | ATTEAN TWP | E | 90 |  |  | S- |  |  |  |  | 2 | AC | UNDEV |  |
| MUD P | 2884 | T06 R12 WELS | E | 1357 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 2908 | T04 R12 WELS | E | 392 |  | S |  |  |  | S |  | 2 | INAC | UNDEV |  |
| MUD P | 2968 | T09 R15 WELS | G | 173 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 3092 | DREW PLT | F | 250 |  | O |  |  |  |  |  | 1B | AC | UNDEV |  |
| MUD P | 3334 | COBURN GORE | D | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUD P | 3538 | RANGELEY PLT | D | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 4056 | MOXIE GORE | D | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 4084 | CHASE STREAM TWP | D | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 5068 | BEATTIE TWP | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUD P | 9917 | JIM POND TWP | D | 14 | S |  |  |  |  |  |  | 2 | AC | DEV | 3 |
| MUD P (LITTLE) | 0738 | T04 R12 WELS | E | 43 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUD P (LITTLE) | 2180 | T06 R08 WELS | F | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MULE BROOK DEADWATER | 9796 | T10 R10 WELS | G | 23 | + |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MULE P | 2422 | RUSSELL POND TWP | E | 14 |  |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| MUNSON L | 1350 | BERRY TWP | C | 40 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| MUNSUNGAN L | 4180 | T08 R10 WELS | G | 1415 | O |  | O | S |  | O |  | 1A | AC | UNDEV | 2 |
| MURPHY P | 0486 | TA R11 WELS | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| MURPHY P (BIG) | 0638 | RAINBOW TWP | E | 15 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| MUSCALSEA P (BIG) | 4036 | RUSSELL POND TWP | E | 14 | m |  | S |  |  |  |  | 2 | INAC | UNDEV | 6 |
| MUSCALSEA P (LITTLE) | 4034 | RUSSELL POND TWP | E | 11 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| MUSKRAT P | 2532 | THORNDIKE TWP | E | 100 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUSQUACOOK L (1ST) | 1914 | T11 R11 WELS | G | 698 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| MUSQUACOOK L (2ND) | 1916 | T11 R11 WELS | G | 813 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| MUSQUACOOK L (3RD) | 1918 | T11 R11 WELS | G | 397 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| MUSQUACOOK L (4TH) | 1920 | T10 R11 WELS | G | 749 | S | S | S |  |  | S |  | 1B | AC | UNDEV |  |
| MUSQUACOOK L (5TH) | 1946 | T10 R11 WELS | G | 358 |  | O | S |  |  | S |  | 1B | AC | UNDEV |  |
| MUSQUASH L (LITTLE) | 1128 | T37 MD BPP | C | 26 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MUSQUASH L (WEST) | 1096 | T06 R01 NBPP | C | 1613 | O |  | O | S |  | S |  | 1A | AC | UNDEV | 2 |
| MUSQUASH P (FIRST) | 0566 | T01 R11 WELS | E | 53 |  |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| MUSQUASH P (SECOND) | 0564 | T01 R11 WELS | E | 55 |  |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| MUSQUASH P (THIRD) | 0570 | T01 R11 WELS | E | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| MYRICK P | 4416 | T10 SD | C | 45 |  |  | S |  |  |  |  | 2 | AC | DEV |  |
| NAHMAKANTA L | 0698 | T01 R11 WELS | E | 1024 | O |  | O | O | O | S |  | 1A | AC | UNDEV | 2 |
| NARRAGUAGUS L | 4414 | T16 MD | C | 426 | S |  | S | S |  | S |  | 1B | INAC | DEV |  |
| NARROW P | 2976 | T08 R14 WELS | E | 151 | S |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| NESOURDNEHUNK DWTR | 0600 | T02 R10 WELS | F | 300 | O | O |  |  |  |  | S | 1A | AC | UNDEV |  |
| NICATOUS L | 4766 | T40 MD | F | 5165 | S | O | O | O |  | S |  | 1A | AC | DEV | 4 |
| NOLLESEMIC L | 2128 | CEDAR LAKE TWP | F | 660 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| NORTH P | 0870 | ELLIOTTSVILLE TWP | E | 58 | S |  | + |  |  |  |  | 2 | INAC | UNDEV |  |
| NORTH P | 1768 | T08 R03 WELS | G | 16 | m |  |  |  |  |  |  | 3 | INAC | DEV |  |
| NORTH P | 3284 | GRAFTON TWP | D | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| NORTH P | 9781 | T14 R09 WELS | G | 15 | O |  |  |  |  | S |  | 1B | INAC | UNDEV | 1 |
| NORTHWEST P | 3342 | MASSACHUSETTS GORE | D | 45 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| NORTHWEST P (LITTLE) | 3344 | MASSACHUSETTS GORE | D | 10 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| NORWAY L | 9526 | PUKAKON TWP | F | 92 |  |  | S | S |  |  |  | 2 | AC | UNDEV |  |
| NOTCH P | 0786 | BOWDOIN COL GR WEST | E | 10 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| NOTCH P (BIG) | 0328 | MOOSEHEAD JUNCTION TWP | E | 12 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| NOTCH P (LITTLE) | 0326 | MOOSEHEAD JUNCTION TWP | E | 10 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| NUMBER NINE L | 1756 | T09 R03 WELS | G | 120 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| NUMBER THREE P | 9635 | T03 R01 NBPP | F | 666 | S |  | S | S | O |  |  | 1B | AC | UNDEV |  |
| OAK KNOLL B DEADWTR | 2010 | CEDAR LAKE TWP | F | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ONAWA L | 0894 | ELLIOTTSVILLE TWP | E | 1344 | O | O | O | S |  | S |  | 1A | AC | DEV | 3 |
| OQIM P (BIG) | 5076 | LOWELLTOWN TWP | E | 97 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| ORIE L | 1094 | T06 R01 NBPP | C | 42 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| OTTER BOG | 4408 | T09 SD | C | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| OTTER L | 1694 | T03 R04 WELS | F | 81 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| OTTER P | 0114 | BOWTOWN TWP | D | 77 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| OTTER P | 1216 | T24 MD BPP | C | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| OTTER P | 2860 | SOPER MOUNTAIN TWP | G | 40 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| OTTER P | 2872 | T08 R14 WELS | E | 109 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| OTTER P | 2924 | T03 R13 WELS | E | 25 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| OTTER P | 3074 | T08 R05 WELS | G | 64 |  |  |  |  | S |  |  | 2 | AC | UNDEV |  |
| OTTER P | 3338 | CHAIN OF PONDS TWP | D | 30 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| OTTER P | 3972 | PARMACHENEE TWP | D | 14 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| OTTER P | 4110 | SANDWICH ACAD GRANT | E | 12 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| OTTER P | 7142 | MAYFIELD TWP | D | 25 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| OTTER P | 7344 | CARRYING PLC TWN TWP | D | 12 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| OTTER P (NORTH) | 0116 | BOWTOWN TWP | D | 71 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| OXBROOK L (LOWER) | 1106 | T06 R01 NBPP | C | 365 | S |  | S | S |  |  |  | 2 | AC | DEV |  |
| OXBROOK L (UPPER) | 1104 | T06 R01 NBPP | C | 422 | S |  | S | S |  |  |  | 2 | AC | UNDEV |  |
| OXHEAD P (LOWER) | 4768 | T40 MD | F | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| OXHEAD P (MIDDLE) | 4770 | T40 MD | F | 38 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| OXHEAD P (UPPER) | 5808 | T40 MD | F | 87 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PACKARD STREAM DWTR | 9584 | LAKE VIEW PLT | F | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PALIN P | 0256 | BRIGHTON PLT | D | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PALMER P | 0266 | MAYFIELD TWP | D | 40 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PALMER P (LITTLE) | 0268 | MAYFIELD TWP | D | 22 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PAPOOSE P | 0338 | MOOSEHEAD JUNCTION TWP | E | 3 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| PAPOOSE P (LITTLE) | 3268 | ALBANY TWP | A | 19 | m |  | S |  |  |  |  | 2 | AC | DEV | 5 |
| PARKER P | 5148 | T03 R05 BKP WKR | D | 80 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PARLIN P | 2544 | PARLIN POND TWP | E | 543 | S |  | S | S | S |  |  | 1B | AC | DEV |  |
| PARMACHENEE L | 3966 | LYNCHTOWN TWP | D | 912 | S | S |  |  |  | S- | O | 1B | AC | UNDEV |  |
| PARTRIDGE P | 2790 | T08 R11 WELS | G | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PARTRIDGE P (LOWER) | 2788 | T08 R11 WELS | G | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PARTRIDGE P (UPPER) | 2792 | T08 R11 WELS | G | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PASSAMAGAMET L | 0970 | T01 R09 WELS | F | 461 |  |  | S | S | O |  |  | 1B | INAC | UNDEV | 1 |
| PATRICK L | 1380 | MARION TWP | C | 275 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PATTE MILL P | 6761 | ALBANY TWP | A | 26 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PATTEN P | 1330 | GRAND LAKE STREAM PLT | C | 96 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PEAKED MOUNTAIN P | 1254 | T19 MD BPP | C | 227 | S |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| PEAKED MOUNTAIN P | 2208 | T04 R07 WELS | F | 61 | S |  | S | S |  |  |  | 2 | INAC | DEV |  |
| PEAKED MOUNTAIN P | 5774 | T10 R11 WELS | G | 13 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PEARL P # 1 | 0416 | BOWDOIN COL GR WEST | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PEEP L | 9821 | T30 MD BPP | C | 32 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| PELLETIER B L (1ST) | 1564 | T16 R09 WELS | G | 25 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PELLETIER B L (2ND) | 1566 | T16 R09 WELS | G | 26 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PELLETIER B L (3RD) | 1560 | T16 R09 WELS | G | 83 | O | + |  |  |  |  |  | 1B | AC | UNDEV |  |
| PELLETIER B L (4TH) | 1562 | T16 R09 WELS | G | 49 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| PELLETIER B L (5TH) | 1504 | T15 R09 WELS | G | 27 | S |  |  |  |  | + |  | 2 | INAC | UNDEV |  |
| PELLETIER B L (6TH) | 1502 | T15 R09 WELS | G | 32 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PEMADUMCOOK CHAIN L* | 0982 | T01 R10 WELS | F | 18300 | S |  | O | S |  | O | S | 1A | AC | DEV | 3 |
| PEMADUMCOOK L* | PPEM | T01 R09 WELS | F | 7356 | S |  | O | S |  | S | S | 1A | AC | UNDEV | 3 |
| PENMAN P | 0113 | T26 ED BPP | C | 29 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| PENNINGTON P | 1612 | T15 R06 WELS | G | 45 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| PENOBSCOT L | 0339 | DOLE BROOK TWP | E | 1019 | O |  | O | S |  | S | O | 1A | AC | UNDEV | 2 |
| PENOBSCOT P | 0568 | T01 R12 WELS | E | 279 | S |  | S |  | O |  |  | 1B | AC | UNDEV |  |
| PENOBSCOT P (LITTLE) | 0560 | T01 R11 WELS | E | 38 |  |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| PEPPERPOT P | 3298 | ADAMSTOWN TWP | D | 50 | m | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| PERCH P | 1522 | T15 R09 WELS | G | 17 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PETE'S P | 1482 | T13 R10 WELS | G | 20 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| PICKEREL L | 1166 | T30 MD BPP | C | 23 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PICKEREL P | 4587 | T32 MD | F | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PICKEREL P (LITTLE) | 1080 | BROOKTON TWP | F | 27 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| PICKEREL P (LITTLE) | 4780 | T34 MD | C | 13 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PICKETT MOUNTAIN P | 3658 | T06 R06 WELS | F | 173 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| PIERCE P | 0086 | PIERCE POND TWP | D | 1650 | O | S | O | S+ |  |  | + | 1A | AC | UNDEV | 2 |
| PIKE BROOK P (EAST) | 9819 | T18 MD BPP | C | 12 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PIKE BROOK P (WEST) | 9667 | T18 MD BPP | C | 32 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PILLSBURY P | 2786 | T08 R11 WELS | G | 301 |  |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| PILLSBURY P (LITTLE) | 2800 | T08 R11 WELS | G | 45 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PINE P | 2990 | T07 R15 WELS | E | 16 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PINE P (BIG) | 2920 | T03 R13 WELS | E | 164 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PINE P (LITTLE) | 9716 | T03 R13 WELS | E | 25 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PINE STREAM FLOWAGE | 2906 | T04 R13 WELS | E | 167 |  | S |  |  | O |  |  | 1B | AC | UNDEV |  |
| PISTOL L (LOWER) | 4756 | T03 ND | F | 979 | S | S | S | S |  |  |  | 1B | AC | UNDEV |  |
| PISTOL L (MIDDLE) | 4750 | OQITON TWP | F | 112 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PISTOL L (SIDE) | 4752 | T03 ND | F | 147 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PISTOL L (UPPER) | 4748 | OQITON TWP | F | 128 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PITMAN P | 0598 | T02 R10 WELS | F | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| PLEASANT & MUD LAKES | 3670 | T06 R06 WELS | F | 498 | S |  | O |  |  |  |  | 1B | AC | UNDEV |  |
| PLEASANT L | 1100 | T06 R01 NBPP | C | 1574 | O |  | O | S | O |  |  | 1A | AC | UNDEV | 2 |
| PLEASANT L | 1728 | T04 R03 WELS | F | 1832 | O |  | S |  |  | S |  | 1B | AC | UNDEV |  |
| PLEASANT L (BIG) | 2756 | T09 R11 WELS | G | 979 | S | S | O | S |  | S |  | 1A | AC | UNDEV |  |
| PLEASANT P | 0224 | THE FORKS PLT | D | 1120 | S |  | + | + |  | S | S | 2 | AC | DEV | 5 |
| PLEASANT P | 2160 | ORNEVILLE TWP | F | 92 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| PLEASANT P (BIG) | 0512 | TA R11 WELS | E | 195 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PLEASANT P (LITTLE) | 0510 | TA R11 WELS | E | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PLEASANT P (LITTLE) | 1943 | T10 R11 WELS | G | 91 | O | S |  | + |  |  |  | 1B | AC | UNDEV |  |
| PLEASANT RIVER L | 1210 | DEVEREAUX TWP | C | 949 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| PLUNKETT P | 3056 | SILVER RIDGE TWP | F | 435 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| POCKWOCKAMUS DEADWTR | 9688 | T02 R09 WELS | F | 275 |  |  |  |  |  |  | S | 2 | INAC | DEV |  |
| POCKWOCKAMUS P | 0245 | T02 R09 WELS | F | 46 |  |  | S | S |  |  | S | 2 | AC | UNDEV |  |
| POCUMCUS L | 1110 | SAKOM TWP | C | 2201 | O | O |  |  |  | S |  | 1A | AC | UNDEV | 3 |
| POLAND P* | 2994 | T07 R14 WELS | E | 490 | S | O | O | S |  |  | O | 1A | AC | UNDEV |  |
| POLAND P (UPPER)* | PPUP | T07 R14 WELS | E | 245 | S | O | O | S |  |  | O | 1A | INAC | UNDEV | 1 |
| POLLY P | 0692 | T03 R11 WELS | E | 15 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| POLLYWOG P | 0576 | T01 R11 WELS | E | 147 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| POND IN THE RIVER | 3328 | TOWNSHIP C | D | 512 | O | S | S |  | O |  |  | 1A | AC | DEV | 4 |
| POOLER P | 4106 | THE FORKS PLT | D | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| PORK BARREL L | 1102 | T06 R01 NBPP | C | 33 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PORTAGE P (LOWER) | 2760 | T09 R11 WELS | G | 35 |  |  | S |  |  | S |  | 2 | AC | UNDEV |  |
| PORTAGE P (UPPER) | 2762 | T09 R11 WELS | G | 96 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| PORTER P | 4760 | T03 ND | F | 58 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| POSSUM P | 1310 | T26 ED BPP | C | 30 | m |  |  |  |  |  |  | 3 | AC | DEV |  |
| PRATT L | 1972 | T11 R09 WELS | G | 96 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| PRENTISS P | 0562 | T01 R11 WELS | E | 12 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| PRESCOTT P | 0898 | ELLIOTTSVILLE TWP | E | 14 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| PRESCOTT P | 4058 | MOXIE GORE | D | 30 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| PRESLEY L | 1870 | T12 R17 WELS | G | 202 |  |  |  |  |  |  | P | 3 | AC | UNDEV |  |
| PRESLEY L (LITTLE) | 1876 | T12 R17 WELS | G | 32 |  |  |  |  |  | S | P | 2 | INAC | UNDEV |  |
| PRESQUE ISLE L | 1758 | T09 R03 WELS | G | 38 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| PRETTY P | 1214 | T24 MD BPP | C | 27 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| PRETTY P | 2802 | T08 R11 WELS | G | 45 |  |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| PRIESTLY L | 1906 | T10 R13 WELS | G | 645 | S |  | O |  |  | S | O | 1A | AC | UNDEV |  |
| PROCTOR P | 3210 | ALBANY TWP | A | 45 | m |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| PRONG P | 9791 | BEAVER COVE | E | 427 | S | O | O | S |  |  |  | 1A | AC | UNDEV |  |
| PUDDING P | 0932 | BARNARD TWP | E | 12 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| PUG (SILVER PUG) L | 1308 | T26 ED BPP | C | 198 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| PUG L (HOSEA) | 1306 | T26 ED BPP | C | 58 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| PUG L (LOWER) | 4694 | LAKEVILLE | F | 70 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| PUG L (UPPER) | 4696 | LAKEVILLE | F | 50 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| PUGHOLE P | 4786 | T41 MD | F | 70 | S |  |  | + | O |  |  | 1B | AC | UNDEV |  |
| PUNCHBOWL P | 0294 | BLANCHARD TWP | E | 40 | S |  | O | S |  |  |  | 1B | AC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PUSHINEER P | 1514 | T15 R09 WELS | G | 55 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| QUAKISH L | 2116 | T03 INDIAN PURCHASE TWP | F | 1000 | S | O |  |  |  | S | S | 1B | AC | UNDEV |  |
| RABBIT P | 0366 | ELLIOTTSVILLE TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| RABBIT P | 0552 | T01 R11 WELS | E | 10 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| RAGGED L | 2936 | T02 R13 WELS | E | 2712 | O |  | O | S |  | S |  | 1A | AC | UNDEV | 4 |
| RAGGED P | 2746 | T09 R10 WELS | G | 11 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| RAGGED P (LOWER E) | 0996 | T04 INDIAN PURCHASE TWP | F | 30 |  |  | S |  |  |  | S | 2 | INAC | UNDEV |  |
| RAGGED P (LOWER W) | 0994 | T04 INDIAN PURCHASE TWP | F | 30 |  |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| RAGGED P (UPPER E) | 0998 | T04 INDIAN PURCHASE TWP | F | 13 |  |  |  |  |  |  | S | 2 | INAC | UNDEV |  |
| RAGGED P (UPPER W) | 0992 | T04 INDIAN PURCHASE TWP | F | 40 |  |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| RAINBOW DEADWATERS | 9698 | RAINBOW TWP | E | 58 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| RAINBOW L | 0614 | RAINBOW TWP | E | 1664 | O |  | O | O |  |  | S | 1A | INAC | UNDEV | 1 |
| RAINBOW P | 4436 | T10 SD | C | 17 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| RANCOURT P | 2632 | DENNISTOWN PLT | E | 23 | S | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| RANGELEY L | 3300 | RANGELEY PLT | D | 6000 | O | S | O | S | S | O | O | 1A | AC | DEV | 4 |
| RAYS MILL P | 0808 | T07 R09 NWP | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| REDINGTON P | 2318 | REDINGTON TWP | D | 37 | S |  | S |  |  |  |  | 2 | AC | DEV |  |
| REED DEADWATER | 2848 | T08 R10 WELS | G | 10 | + | S |  |  |  |  |  | 3 | INAC | UNDEV |  |
| REED P | 3088 | MACWAHOC PLT | F | 19 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| REED P (BIG) | 2842 | T08 R10 WELS | G | 90 | O |  |  |  | O |  |  | 1A | INAC | UNDEV | 1 |
| REED P (LITTLE) | 2838 | T08 R10 WELS | G | 25 | m+ |  |  |  | + |  |  | 3 | INAC | UNDEV | 6 |
| RICHARDSON L* | 3308 | RICHARDSONTOWN TWP | D | 7100 | O | O | O | O |  | O | S | 1A | AC | DEV |  |
| RICHARDSON L (LOWER)* | RHLW | TOWNSHIP C | D | 2900 | O | S | S | O |  | S | S | 1A | AC | DEV | 3 |
| RICHARDSON L (UPPER)* | RHUP | RICHARDSONTOWN TWP | D | 4200 | O | O | O | O |  | O |  | 1A | AC | DEV |  |
| RICHARDSON P (LO E) | 3296 | ADAMSTOWN TWP | D | 54 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| RICHARDSON P (UP E) | 3294 | ADAMSTOWN TWP | D | 85 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| RICHARDSON P (WEST) | 3292 | ADAMSTOWN TWP | D | 423 |  | S | O | S |  |  |  | 1B | AC | DEV |  |
| RIPOGENUS P | 2910 | T04 R12 WELS | E | 76 | m | S |  |  |  | S |  | 2 | INAC | UNDEV | 6 |
| RIVER L (LITTLE) | 1118 | T43 MD BPP | C | 75 | m+ | S | + |  |  |  |  | 2 | INAC | UNDEV |  |
| RIVER P | 2092 | T02 R09 WELS | F | 125 |  |  | S |  |  |  | S | 2 | AC | UNDEV |  |
| ROACH P (FIRST) | 0436 | FRENCHTOWN TWP | E | 3270 | S |  | S | S | S | S |  | 1B | AC | DEV | 3 |
| ROACH P (FOURTH) | 0446 | SHAWTOWN TWP | E | 266 | S |  | O | S |  |  |  | 1B | INAC | UNDEV | 1/6 |
| ROACH P (SECOND) | 0452 | T01 R12 WELS | E | 970 | S |  | S |  |  | S | S | 1B | AC | UNDEV |  |
| ROACH P (SEVENTH) | 0500 | TA R11 WELS | E | 33 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| ROACH P (SIXTH) | 0480 | SHAWTOWN TWP | E | 48 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| ROACH P (THIRD) | 0482 | SHAWTOWN TWP | E | 570 | S |  | O | S |  |  |  | 1B | AC | UNDEV |  |
| ROARING BROOK P | 0918 | T07 R09 NWP | E | 55 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROBAR P | 2014 | T04 R08 WELS | F | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROBAR P (BIG) | 2296 | T04 R08 WELS | F | 7 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| ROBBINS BROOK P | 9794 | T12 R11 WELS | G | 27 | O |  |  |  |  |  |  | 1B | INAC | DEV |  |
| ROBERTS P | 5164 | T05 R20 WELS | E | 19 | m |  |  |  |  |  |  | 3 | AC | UNDEV | 6 |
| ROBINSON P | 2436 | T04 R17 WELS | E | 34 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROCK P | 3340 | CHAIN OF PONDS TWP | D | 26 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ROCK P | 3542 | SANDY RIVER PLT | D | 7 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROCK P | 5108 | HAYNESTOWN TWP | E | 124 | O |  | S+ | S |  |  |  | 1B | AC | UNDEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ROCKABEMA L | 3636 | MORO PLT | G | 339 | S |  | S | S |  |  |  | 2 | AC | DEV | 3 |
| ROCKY BOG | 2008 | CEDAR LAKE TWP | F | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROCKY L | 1348 | BERRY TWP | C | 1555 | O | S |  |  |  |  | O | 1A | AC | UNDEV |  |
| ROCKY P | 0676 | T03 R11 WELS | E | 16 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROCKY P | 1182 | T25 MD BPP | C | 32 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ROCKY P | 2018 | T03 R08 WELS | F | 16 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| ROCKY P | 4476 | T22 MD | C | 666 | m |  |  |  |  |  |  | 3 | AC | UNDEV | 3 |
| ROCKY P (BIG) | 0522 | TA R11 WELS | E | 115 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| ROCKY P (LITTLE) | 0524 | TA R11 WELS | E | 12 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| RODERIQUE P | 0317 | ROCKWOOD STRIP T02 R01 | E | 44 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ROLAND P | 0031 | DALLAS PLT | D | 26 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROSS L | 1888 | T10 R15 WELS | G | 2892 | S |  |  | + |  | S | S | 2 | AC | UNDEV |  |
| ROUND L | 1240 | T19 ED BPP | C | 352 | S | O |  |  |  |  |  | 1B | AC | DEV |  |
| ROUND MOUNTAIN P | 1966 | T11 R08 WELS | G | 38 | S |  |  |  |  | O |  | 1B | AC | UNDEV |  |
| ROUND MOUNTAIN P | 5058 | ALDER STREAM TWP | D | 73 | S+ |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| ROUND P | 0235 | T03 R01 NBPP | F | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ROUND P | 0563 | T10 SD | C | 205 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ROUND P | 1470 | T13 R12 WELS | G | 697 | O | O* |  |  |  | S |  | 1A | AC | UNDEV | 2 |
| ROUND P | 1594 | T14 R08 WELS | G | 90 | S | S |  |  |  |  |  | 2 | AC | UNDEV | 3 |
| ROUND P | 2670 | APPLETON TWP | E | 5 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| ROUND P | 2914 | LOBSTER TWP | E | 93 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ROUND P | 3584 | TOWNSHIP E | D | 42 | O |  |  |  |  |  |  | 1B | INAC | DEV | 5 |
| ROUND P | 4004 | T07 R14 WELS | E | 375 | S |  | S | S |  |  |  | 2 | AC | UNDEV |  |
| ROUND P | 4076 | SQUARETOWN TWP | D | 60 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| ROUND P | 4092 | CHASE STREAM TWP | D | 30 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| ROUND P | 4100 | SAPLING TWP | E | 40 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROUND P | 4158 | T07 R09 WELS | G | 30 |  | + | + |  |  |  | + | 3 | INAC | UNDEV |  |
| ROUND P | 6755 | ALBANY TWP | A | 14 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| ROUND P (LITTLE) | 2874 | EAGLE LAKE TWP | G | 58 | O | S |  |  |  |  | O | 1A | INAC | UNDEV | 1 |
| ROUND P (LITTLE) | 4382 | T07 SD | C | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ROWE L | 1964 | T11 R08 WELS | G | 252 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ROWE P | 0202 | PLEASANT RIDGE PLT | D | 205 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ROWE P | 4002 | T07 R15 WELS | E | 250 | S |  | O | S |  |  | S | 1B | AC | UNDEV |  |
| RUM P | 0780 | BOWDOIN COL GR WEST | E | 245 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| RUMP P | 3112 | PARMACHENEE TWP | D | 35 | S |  |  |  |  | S- |  | 2 | AC | DEV |  |
| RUSH P | 3062 | HERSEYTOWN TWP | F | 243 |  | O |  |  |  |  | S | 1B | AC | UNDEV |  |
| RUSH P | 5130 | KING & BARTLETT TWP | D | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| RUSSELL P | 2424 | RUSSELL POND TWP | E | 152 | S |  | S | S |  |  |  | 2 | AC | UNDEV |  |
| RUSSELL P (LOWER) | 2852 | T09 R14 WELS | G | 140 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| RUSSELL P (MIDDLE) | 2958 | T09 R14 WELS | G | 24 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| RUSSELL P (UPPER) | 2960 | T09 R14 WELS | G | 282 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| RUSSELL P(1ST SO BR) | 2978 | T08 R14 WELS | E | 40 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| RUSSELL P(2ND SO BR) | 2988 | T08 R14 WELS | E | 66 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SABAO L (LOWER) | 4784 | T35 MD | C | 755 | O |  |  |  |  | S |  | 1B | AC | UNDEV |  |
| SABAO L (UPPER) | 4522 | T41 MD | F | 486 | S |  | + | + | O | S | S | 1B | AC | UNDEV |  |
| SABBATH DAY P | 3578 | TOWNSHIP E | D | 57 | S |  |  |  |  |  |  | 2 | INAC | DEV |  |
| SADDLE P | 4240 | T07 R09 WELS | F | 64 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| SADDLEBACK L | 3536 | DALLAS PLT | D | 358 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| SADDLEBACK P | 3550 | SANDY RIVER PLT | D | 13 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| SADDLEROCK P | 9662 | TB R11 WELS | F | 10 |  |  |  |  |  |  | S | 2 | AC | UNDEV |  |
| SAFFORD P | 0006 | LEXINGTON TWP | D | 40 |  |  |  |  | O |  |  | 1B | AC | UNDEV |  |
| SAG P | 1484 | T13 R10 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SAINT CROIX L | 1774 | ST CROIX TWP | G | 416 | S | S |  |  |  |  | S | 2 | AC | DEV |  |
| SAINT FRANCIS L | 2398 | T08 R16 WELS | G | 322 |  |  |  |  |  |  | P | 3 | AC | UNDEV |  |
| SAINT FROID L | 1610 | WINTERVILLE PLT | G | 2400 | S |  |  | + |  | S | S | 2 | AC | DEV |  |
| SALMON P | 1158 | T30 MD BPP | C | 11 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| SALMON P | 2900 | T04 R13 WELS | E | 107 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SALMON P | 4422 | T10 SD | C | 10 | S |  | + |  |  |  |  | 2 | INAC | UNDEV |  |
| SALMON STREAM L | 3046 | T01 R06 WELS | F | 659 |  |  | S |  |  |  | S | 2 | INAC | UNDEV |  |
| SALMON STREAM L (LT) | 3048 | T01 R06 WELS | F | 218 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SAM HILL L | 1156 | DAY BLOCK TWP | C | 46 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SAMPSON P | 0812 | T07 R09 NWP | E | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SANDY RIVER P (MID) | 3566 | SANDY RIVER PLT | D | 70 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| SANDY RIVER P(LOWER) | 3564 | SANDY RIVER PLT | D | 17 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| SANDY RIVER P(UPPER) | 3568 | SANDY RIVER PLT | D | 28 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| SAPET P (LITTLE) | 5078 | LOWELLTOWN TWP | E | 22 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SAPONAC P | 4722 | GRAND FALLS TWP | F | 922 | S |  | S | S |  | S | P- | 1B | AC | DEV | 3 |
| SAWTELLE DEADWATER | 2174 | T06 R07 WELS | F | 218 | S | O | S |  |  |  |  | 1B | AC | UNDEV |  |
| SAWTELLE P | 3008 | T07 R08 WELS | F | 174 |  | O |  |  |  |  |  | 1B | INAC | UNDEV | 1 |
| SAWTELLE P (LITTLE) | 5778 | T07 R08 WELS | F | 10 |  | O |  |  |  |  |  | 1B | INAC | UNDEV | 1 |
| SCHOODIC L | 0956 | LAKE VIEW PLT | F | 7168 | S |  | S |  |  | S | S | 1B | AC | DEV | 3,5 |
| SCHOODIC L | 1230 | T18 MD BPP | C | 389 | S |  |  |  | O |  |  | 1B | AC | UNDEV |  |
| SCOPAN L | 1654 | SCOPAN TWP | G | 5120 | S |  |  |  | S | S | P | 2 | AC | DEV |  |
| SCOTT DEADWATER (LT) | 4010 | T06 R14 WELS | E | 24 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SCOTT P | 1762 | T08 R03 WELS | G | 12 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SCOTT P (LITTLE) | 4030 | T05 R14 WELS | E | 47 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| SCRAGGLEY L | 9649 | T06 R01 NBPP | C | 2758 | S |  | S | S |  | S |  | 1B | AC | UNDEV |  |
| SCRAGGLY L | 4264 | T07 R08 WELS | F | 842 | O |  | O | O | O | S | O | 1A | AC | UNDEV | 2 |
| SCRIBNER BOG | 4072 | SQUARETOWN TWP | D | 15 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| SCUTAZE STREAM DWTR | 9586 | LAKE VIEW PLT | F | 22 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SEARS P | 9766 | DOLE BROOK TWP | E | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SEBOEIS DEADWATER | 2172 | T06 R07 WELS | F | 60 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SEBOEIS L | 0954 | T04 R09 NWP | F | 4201 | S | S | O | S |  | S | S | 1A | AC | UNDEV |  |
| SEBOOMOOK L | 4048 | PLYMOUTH TWP | E | 6448 | S | O | S | S | S | S | O | 1A | AC | UNDEV |  |
| SECOND L | 1134 | T37 MD BPP | C | 102 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| SECOND L | 1268 | BERRY TWP | C | 332 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SECOND L | 1374 | MARION TWP | C | 1650 | S | O |  |  |  |  | O | 1A | AC | UNDEV |  |
| SECRET P | 0907 | ELLIOTTSVILLE TWP | E | 12 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| SECRET P | 3358 | SEVEN PONDS TWP | D | 10 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SELMORE P | 4738 | OQITON TWP | F | 22 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SEWALL DEADWATER P | 2748 | T09 R10 WELS | G | 14 | + | S | + |  |  |  |  | 3 | AC | UNDEV |  |
| SHACK P | 2912 | LOBSTER TWP | E | 69 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SHAD P | 2120 | T03 INDIAN PURCHASE TWP | F | 112 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SHADOW P | 0331 | MOOSEHEAD JUNCTION TWP | E | 17 | S |  |  |  |  |  | S | 2 | AC | UNDEV |  |
| SHALLOW L | 2876 | T07 R14 WELS | E | 1110 | m+ | O |  |  |  |  |  | 1B | AC | UNDEV |  |
| SHALLOW L (LITTLE) | 2868 | T07 R14 WELS | E | 308 | m |  |  |  |  | S | P | 2 | INAC | UNDEV |  |
| SHALLOW P | 5052 | JIM POND TWP | D | 52 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| SHAW L | 7407 | T06 R01 NBPP | C | 211 | S |  | S |  |  |  |  | 2 | INAC | UNDEV |  |
| SHAW P | 5152 | T03 R04 BKP WKR | D | 45 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| SHAW P (LOWER) | 5154 | T03 R04 BKP WKR | D | 64 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| SHILLALAH P | 4440 | T10 SD | C | 21 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SHIN P (LOWER) | 2198 | T05 R07 WELS | F | 638 | S |  | S |  |  | S |  | 2 | AC | DEV | 5 |
| SHIN P (UPPER) | 2202 | MOUNT CHASE | F | 544 | O |  | S |  |  | S |  | 1B | AC | DEV |  |
| SHINY L | 7433 | MARION TWP | C | 32 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SHIRLEY BOG (EAST) | 9600 | MOOSEHEAD JUNCTION TWP | E | 70 |  |  |  |  |  |  | S | 2 | AC | UNDEV |  |
| SHIRLEY BOG (WEST) | 0350 | MOOSEHEAD JUNCTION TWP | E | 275 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SILVER L | 0922 | KATAHDIN IRN WKS TWP | F | 305 | S |  | S | S |  |  | S | 1B | AC | DEV | 3 |
| SILVER L | 1656 | T15 R05 WELS | G | 13 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SIMON P | 1324 | CODYVILLE PLT | C | 14 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SIMSQUISH L | 1316 | LAMBERT LAKE TWP | C | 115 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SING SING P | 0558 | T01 R11 WELS | E | 50 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SKITACOOK L | 1730 | T04 R03 WELS | F | 435 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SLAUGHTER P | 0690 | T03 R11 WELS | E | 66 | O | + | O | S+ |  | S | + | 1B | INAC | UNDEV | 1/6 |
| SMITH (WEEKS) P | 0254 | BRIGHTON PLT | D | 160 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SMITH BROOK P | 1770 | DUDLEY TWP | G | 32 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SMITH BROOK P | 4184 | T09 R10 WELS | G | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SMITH P | 0896 | ELLIOTTSVILLE TWP | E | 10 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SMITH P | 2012 | T03 INDIAN PURCHASE TWP | F | 208 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| SMITH P | 2546 | PARLIN POND TWP | E | 16 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SMITH P | 2638 | FORSYTH TWP | E | 15 | m |  |  |  |  |  | S | 2 | INAC | UNDEV |  |
| SMITH P | 3078 | T08 R05 WELS | G | 32 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SMITH P (LITTLE) | 2112 | T01 R08 WELS | F | 24 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SNAKE P | 2548 | JOHNSON MOUNTAIN TWP | E | 8 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| SNAKE P | 2702 | T07 R11 WELS | E | 275 | S |  | S | S |  |  |  | 2 | INAC | UNDEV |  |
| SNOW MOUNTAIN P | 5060 | ALDER STREAM TWP | D | 12 | S |  |  |  |  |  |  | 2 | INAC | DEV |  |
| SNOWSHOE L | 3028 | T07 R07 WELS | F | 638 | S |  | O |  |  |  |  | 1B | AC | UNDEV |  |
| SNOWSHOE P | 2804 | T08 R11 WELS | G | 40 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SOCATEAN P #1 | 4044 | PLYMOUTH TWP | E | 42 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| SOCATEAN P #2 | 4046 | PLYMOUTH TWP | E | 14 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| SOLDIER P | 2314 | SOLDIERTOWN T02 R07 WELS | F | 42 |  |  |  |  |  |  | S | 2 | INAC | UNDEV |  |
| SOLDIER P | 9783 | WALLAGRASS | G | 96 | S |  |  |  |  |  |  | 2 | AC | DEV | 5 |
| SOLDIER P (LITTLE) | 2308 | SOLDIERTOWN T02 R07 WELS | F | 20 |  |  | S |  |  |  | S | 2 | INAC | UNDEV |  |
| SONGO P | 3262 | ALBANY TWP | A | 224 | S |  |  |  | S | S |  | 2 | AC | DEV | 5 |
| SOPER BROOK DWTR | 9674 | T04 R11 WELS | E | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SOPER LOGAN | 2184 | T06 R08 WELS | F | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SOPER P | 2782 | SOPER MOUNTAIN TWP | G | 246 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SOPER P (UPPER) | 2784 | T08 R11 WELS | G | 76 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SOULE P | 4166 | T07 R08 WELS | F | 19 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SOURDNAHUNK L | 2730 | T05 R11 WELS | E | 1394 | O |  | S | S |  |  |  | 1B | AC | UNDEV |  |
| SOURDNAHUNK L (LIT) | 2728 | T05 R11 WELS | E | 102 | O |  | S |  |  |  |  | 1B | AC | UNDEV |  |
| SOUTH (PINE TREE) P | 3560 | SANDY RIVER PLT | D | 25 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SOUTH P | 0824 | T07 R09 NWP | E | 18 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SPAULDING P | 9764 | DOLE BROOK TWP | E | 28 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SPECK P | 3288 | GRAFTON TWP | D | 9 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| SPECTACLE (SPEC) P | 4450 | OSBORN | C | 1754 | O |  |  |  |  |  |  | 1B | AC | DEV | 3 |
| SPECTACLE L (LOWER) | 1342 | T19 ED BPP | C | 26 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SPECTACLE L (UPPER) | 1344 | T19 ED BPP | C | 26 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SPECTACLE P | 0282 | BLANCHARD TWP | E | 60 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SPECTACLE P | 1580 | T10 R08 WELS | G | 122 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| SPECTACLE P | 5122 | KING & BARTLETT TWP | D | 45 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| SPEDNIK L | 0121 | FOREST CITY TWP | F | 17219 | S | O | + |  |  | S+ | + | 1B | AC | UNDEV |  |
| SPENCER L | 5104 | HOBBSTOWN TWP | E | 1819 | O |  | O | O | O | O |  | 1A | AC | UNDEV | 2 |
| SPENCER P | 0404 | E MIDDLESEX CANAL GR | E | 980 | S | O* | O | S |  |  |  | 1A | AC | UNDEV | 2 |
| SPENCER P | 2450 | COMSTOCK TWP | E | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SPENCER P | 3586 | TOWNSHIP D | D | 15 | S |  |  |  |  |  |  | 2 | INAC | DEV |  |
| SPENCER P | 4754 | T03 ND | F | 38 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SPENCER P (LITTLE) | 2950 | E MIDDLESEX CANAL GR | E | 75 |  | S |  |  | O |  |  | 1B | AC | UNDEV |  |
| SPIDER L | 2758 | T09 R11 WELS | G | 890 | S |  | O | S |  | S |  | 1B | AC | UNDEV |  |
| SPRING L | 0170 | T03 R04 BKP WKR | D | 762 | S | S |  |  |  | S |  | 2 | AC | DEV |  |
| SPRING L | 4758 | T03 ND | F | 435 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| SPRING P | 2170 | T06 R07 WELS | F | 17 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SPRING P | 2832 | T07 R10 WELS | G | 15 | O | + | + |  |  |  |  | 2 | INAC | UNDEV | 6 |
| SPRING RIVER L | 4432 | T10 SD | C | 704 | S |  | O | O |  |  |  | 1A | AC | DEV | 4 |
| SPRUANCE P | 0508 | TA R11 WELS | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SPRUCE MOUNTAIN P | 0466 | TB R11 WELS | F | 20 | S |  | O |  |  |  | S | 1B | INAC | UNDEV | 1/6 |
| SPRUCE P | 0052 | LEXINGTON TWP | D | 49 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| SQUARE L | 1672 | T16 R05 WELS | G | 8150 | O |  |  | + |  | S | S+ | 1B | AC | DEV |  |
| SQUIRREL P | 1932 | T11 R10 WELS | G | 51 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SQUIRTGUN FLOWAGE | 7754 | CHASE STREAM TWP | D | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| ST JOHN L (LITTLE) | 5168 | T05 R20 WELS | E | 90 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| ST JOHN P (FIFTH) | 2414 | T05 R17 WELS | E | 1208 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| ST JOHN P (FOURTH) | 2416 | T05 R17 WELS | E | 198 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| ST JOHN P (SECOND) | 2432 | T04 R17 WELS | E | 105 |  |  |  |  | + |  |  | 3 | INAC | UNDEV | 6 |
| ST JOHN P (THIRD) | 2438 | T04 R17 WELS | E | 190 | S |  |  |  | + |  |  | 2 | INAC | UNDEV | 6 |
| ST JOHN P(LOWER 1ST) | 2428 | T04 R17 WELS | E | 29 |  |  |  |  | + |  |  | 3 | INAC | UNDEV | 6 |
| ST JOHN P(UPPER 1ST) | 2440 | T04 R17 WELS | E | 30 |  |  |  |  | + |  |  | 3 | INAC | UNDEV | 6 |
| STERLING P | 1574 | T13 R07 WELS | G | 38 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| STILES L | 9659 | T35 MD | C | 51 | m+ |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| STINK P | 1518 | T15 R09 WELS | G | 16 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| STINK P & SNAKE BOG | 2818 | T07 R11 WELS | E | 35 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| STONY BROOK P | 0138 | LOWER ENCHANTED TWP | D | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| STRATTON BROOK P | 2317 | WYMAN TWP | D | 26 |  | O | O |  |  |  |  | 1A | AC | UNDEV |  |
| STRATTON P | 0618 | RAINBOW TWP | E | 15 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| STURTEVANT P | 3104 | MAGALLOWAY TWP | D | 518 | S |  | O | S |  |  |  | 1B | AC | DEV |  |
| SUCKER BROOK P | 0924 | KATAHDIN IRN WKS TWP | F | 22 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SUGAR BERTH P | 2634 | DENNISTOWN PLT | E | 23 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| SUMMIT P | 2434 | T04 R17 WELS | E | 52 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SUNDAY P | 3316 | MAGALLOWAY TWP | D | 30 | S | S |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| SUNDAY P | 7674 | LYNCHTOWN TWP | D | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SUNKEN & ROCKY LAKES | 1368 | MARION TWP | C | 1126 |  | S |  |  |  |  | O | 1B | AC | UNDEV |  |
| SUNKEN P | 3267 | ALBANY TWP | A | 26 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SUNSET P | 0892 | ELLIOTTSVILLE TWP | E | 10 | m |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| SWEENEY BOG | 9776 | T06 R17 WELS | E | 40 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| SWIFT RIVER P | 3576 | TOWNSHIP E | D | 10 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| SWIFT RIVER P (LIT) | 3572 | TOWNSHIP E | D | 15 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| SYSLADOBSIS L (LO) | 4730 | SAKOM TWP | F | 5376 | S |  | S | S+ | O | S |  | 1A | AC | DEV | 4 |
| SYSLADOBSIS L (UP) | 4688 | LAKEVILLE | F | 1142 | S |  | S | S |  | S |  | 1B | AC | DEV |  |
| TACK P | 2134 | CEDAR LAKE TWP | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| TARBOX P | 4008 | T06 R14 WELS | E | 54 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| TEA P | 2350 | JIM POND TWP | D | 90 | S | S |  |  |  |  | O | 1B | AC | DEV |  |
| TELOS L & ROUND P | 2710 | T06 R11 WELS | E | 2276 | O | S | O | S |  | S |  | 1A | AC | UNDEV | 2 |
| TEN THOUSAND ACRE P | 4088 | CHASE STREAM TWP | E | 37 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| TEN-FORTY P | 9704 | T02 R12 WELS | E | 25 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| TENMILE L | 1058 | FORKSTOWN TWP | F | 45 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| THANKSGIVING P | 0288 | BLANCHARD TWP | E | 17 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| THE HORNS POND | 8601 | WYMAN TWP | D | 10 | S |  | O | O |  |  |  | 1A | INAC | UNDEV | 1 |
| THIRD L | 1132 | T37 MD BPP | C | 141 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| THIRD L | 2704 | T07 R10 WELS | F | 474 | S | + | S+ | S |  |  | + | 2 | INAC | UNDEV |  |
| THIRD L | 8220 | T28 MD | C | 12 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| THISSELL P | 2726 | T05 R11 WELS | E | 141 | S |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| THOMPSON DEADWATER | 1060 | REED PLT | F | 50 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| TILDEN P | 4418 | T10 SD | C | 36 | S |  | + |  |  |  |  | 2 | INAC | UNDEV | 6 |
| TIM P | 2362 | TIM POND TWP | D | 320 | O |  | O |  |  | + |  | 1A | AC | UNDEV | 2 |
| TITCOMB P | 4582 | T32 MD | F | 38 | S |  |  |  |  |  |  | 2 | INAC | DEV |  |
| TOBEY P | 4078 | JOHNSON MOUNTAIN TWP | E | 20 | O |  |  |  |  |  |  | 1B | AC | UNDEV |  |
| TOBEY P #1 | 2674 | T05 R07 BKP WKR | E | 35 | m |  | O | S |  |  |  | 1B | INAC | UNDEV | 1/6 |
| TOBEY P #2 | 2676 | T05 R07 BKP WKR | E | 32 | m |  | S |  |  |  |  | 2 | INAC | UNDEV | 6 |
| TOBEY P #3 | 2678 | T05 R07 BKP WKR | E | 14 | m |  | S | S |  |  |  | 2 | INAC | UNDEV | 6 |
| TOBY P | 5102 | HOBBSTOWN TWP | D | 28 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| TOGUE P | 1530 | T15 R09 WELS | G | 388 | O |  | S | + |  |  |  | 1B | AC | UNDEV |  |
| TOGUE P (LOWER) | 2084 | T02 R09 WELS | F | 384 | S |  | O | S |  |  | O | 1A | AC | DEV | 4 |
| TOGUE P (UPPER) | 2104 | T02 R09 WELS | F | 294 | S |  | O | S |  |  | O | 1A | AC | DEV | 4 |
| TOM YOUNG P | 2952 | E MIDDLESEX CANAL GR | E | 25 |  | S |  |  |  |  | S | 2 | AC | UNDEV |  |
| TOMAH L | 0135 | FOREST TWP | F | 56 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| TOMAH L (LITTLE) | 1320 | CODYVILLE PLT | C | 147 |  |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| TOMHEGAN P | 4038 | W MIDDLESEX CANAL GR | E | 356 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| TOTE ROAD P | 0425 | MORO PLT | G | 25 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| TRICKEY P | 2514 | ALDER BROOK TWP | E | 23 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| TROUT L | 1098 | KOSSUTH TWP | F | 5 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| TROUT L | 1250 | DAY BLOCK TWP | C | 21 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| TROUT P | 0322 | MOOSEHEAD JUNCTION TWP | E | 33 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| TROUT P | 0448 | SHAWTOWN TWP | E | 145 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| TROUT P | 0792 | BOWDOIN COL GR WEST | E | 20 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| TROUT P | 2316 | SOLDIERTOWN T02 R07 WELS | F | 19 | S |  | S |  |  |  | S | 2 | AC | UNDEV |  |
| TROUT P | 3260 | MASON TWP | A | 17 | m |  | S |  | O |  |  | 1B | INAC | UNDEV | 1/6 |
| TROUT P | 4724 | GRAND FALLS TWP | F | 15 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| TROUT P | 5082 | LOWELLTOWN TWP | E | 55 | m+ |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| TRUESDALE P | 2452 | COMSTOCK TWP | E | 46 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| TRUEWORTHY P | 4762 | T03 ND | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| TUMBLEDOWN DICK P | 0548 | T01 R11 WELS | E | 24 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| TUMBLEDOWN P | 3512 | TOWNSHIP 6 N OF WELD | D | 9 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| TUNK L | 4434 | T10 SD | C | 2010 | O | O | O | O |  | S | S | 1A | AC | DEV | 4 |
| TURNER P | 2402 | T07 R16 WELS | E | 104 | S |  |  |  |  |  | P | 2 | AC | UNDEV |  |
| TURNER P (BIG) | 2642 | FORSYTH TWP | E | 111 | O |  | + |  |  |  | S | 1B | AC | UNDEV |  |
| TURNER P (LITTLE) | 2640 | FORSYTH TWP | E | 77 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| TURTLE P | 0952 | LAKE VIEW PLT | F | 81 | O |  |  |  |  |  |  | 1B | INAC | UNDEV | 1/6 |
| TWIN (TROUT) PONDS | 2102 | T02 R09 WELS | F | 60 | O |  | O | S |  |  |  | 1A | INAC | UNDEV | 1/6 |
| TWIN ISLAND P | 5084 | LOWELLTOWN TWP | E | 84 |  |  | + |  |  |  |  | 3 | AC | UNDEV |  |
| TWIN L (NORTH)* | PNTW | T04 INDIAN PURCHASE TWP | F | 3347 | S |  | O | S |  | S | S | 1A | AC | DEV | 3 |
| TWIN L (SOUTH)* | PSTW | T04 INDIAN PURCHASE TWP | F | 3406 | S |  | O | S |  | S |  | 1B | AC | DEV | 3,5 |
| TWIN P | 1908 | T12 R12 WELS | G | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| TWIN P #1 | 2210 | T04 R07 WELS | F | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| TWO MILE P | 9765 | T16 R13 WELS | G | 12 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UGH L | 1884 | T12 R14 WELS | G | 79 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| UMBAGOG L | 3102 | MAGALLOWAY TWP | D | 7850 | S | O | + | + |  | S+ | + | 1B | AC | UNDEV |  |
| UMBAZOOKSUS L | 2890 | T06 R13 WELS | E | 1590 | O | S |  |  |  | S+ |  | 1B | AC | UNDEV |  |
| UMCOLCUS L | 3080 | T07 R05 WELS | G | 630 | S | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| UMSASKIS L | 1896 | T11 R13 WELS | G | 1222 | O | O* |  |  |  | S | S | 1A | AC | UNDEV | 2 |
| UNKNOWN L (LOWER) | 4740 | OQITON TWP | F | 179 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| UNKNOWN L (MIDDLE) | 4742 | OQITON TWP | F | 90 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| UNKNOWN L (UPPER) | 4744 | OQITON TWP | F | 58 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNKNOWN P | 5072 | GORHAM GORE | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 2879 | T03 R04 WELS | F | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 6946 | T03 R05 BKP WKR | D | 15 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| UNNAMED P | 6948 | T03 R05 BKP WKR | D | 12 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| UNNAMED P | 6950 | T03 R05 BKP WKR | D | 20 |  | S |  |  |  |  |  | 2 | AC | UNDEV |  |
| UNNAMED P | 7016 | KING & BARTLETT TWP | D | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7020 | T12 R16 WELS | G | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7023 | T11 R17 WELS | G | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7062 | THE FORKS PLT | D | 10 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| UNNAMED P | 7066 | THE FORKS PLT | D | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7073 | T06 R15 WELS | E | 8 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UNNAMED P | 7115 | COMSTOCK TWP | E | 15 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UNNAMED P | 7134 | MAYFIELD TWP | D | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7314 | HIGHLAND PLT | D | 12 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| * This lake has multiple management classes corresponding to sections of the lake. Refer to the official zoning map for designations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UNNAMED P | 7319 | T37 MD BPP | C | 12 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7320 | LEXINGTON TWP | D | 10 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| UNNAMED P | 7335 | T30 MD BPP | C | 10 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7379 | T19 MD BPP | C | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7389 | T18 MD BPP | C | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7390 | SQUARETOWN TWP | D | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7397 | FOREST TWP | F | 40 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7403 | T08 R04 NBPP | F | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7421 | FOWLER TWP | C | 32 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7462 | WEST FORKS PLT | D | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7485 | CODYVILLE PLT | C | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7498 | LOWER ENCHANTED TWP | D | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7594 | C SURPLUS | D | 35 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7596 | TOWNSHIP C | D | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7700 | MAGALLOWAY TWP | D | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7752 | CHASE STREAM TWP | D | 11 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 7776 | APPLETON TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7818 | CHASE STREAM TWP | D | 10 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| UNNAMED P | 7862 | SEBOOMOOK TWP | E | 22 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7864 | SEBOOMOOK TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 7996 | T07 R15 WELS | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8038 | T01 R09 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8062 | T01 R10 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8120 | T02 R09 WELS | F | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8180 | T02 R09 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8251 | T41 MD | F | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8289 | T28 MD | C | 10 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8308 | T04 R12 WELS | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8312 | T04 R15 WELS | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8314 | T05 R11 WELS | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8343 | T10 SD | C | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8356 | DOLE BROOK TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8359 | FLETCHER’S LANDING TWP | C | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8363 | FLETCHER’S LANDING TWP | C | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8369 | T07 SD | C | 37 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8381 | T03 ND | F | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8385 | T03 ND | F | 19 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8416 | COMSTOCK TWP | E | 20 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UNNAMED P | 8620 | T04 R17 WELS | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8650 | T05 R17 WELS | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8735 | SALEM TWP | D | 40 |  |  |  |  |  |  |  | 3 | AC | DEV | 5 |
| UNNAMED P | 8737 | RANGELEY PLT | D | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 8807 | FREEMAN TWP | D | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8874 | BRADSTREET TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8886 | BRADSTREET TWP | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8888 | BRADSTREET TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8890 | SOLDIERTOWN T02 R03 NBKP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8918 | FORSYTH TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8934 | ATTEAN TWP | E | 5 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UNNAMED P | 8942 | HOLEB TWP | E | 2 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UNNAMED P | 8958 | MISERY TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8976 | T05 R07 BKP WKR | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 8980 | T05 R07 BKP WKR | E | 10 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UNNAMED P | 9005 | T05 R08 WELS | F | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9007 | T05 R08 WELS | F | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9013 | T05 R07 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9017 | T05 R07 WELS | F | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9019 | T05 R07 WELS | F | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9039 | T04 R07 WELS | F | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9061 | T04 R08 WELS | F | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9067 | T04 R08 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9069 | T04 R08 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9071 | T04 R08 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9081 | T03 R08 WELS | F | 25 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9085 | T03 R08 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9097 | PRENTISS TWP T07 R03 NBPP | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9099 | PRENTISS TWP T07 R03 NBPP | F | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9102 | KATAHDIN IRN WKS TWP | F | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9107 | KINGMAN TWP | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9109 | DREW PLT | F | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9111 | T03 R04 WELS | F | 13 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9113 | T03 R04 WELS | F | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9130 | TB R10 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9137 | TD R02 WELS | G | 10 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| UNNAMED P | 9150 | TA R11 WELS | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9201 | BIG TWENTY TWP | G | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9207 | T18 R12 WELS | G | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9220 | T02 R08 WELS | F | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9229 | T14 R15 WELS | G | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9232 | T08 R15 WELS | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9245 | T13 R15 WELS | G | 51 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9263 | T16 R09 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9276 | GRINDSTONE TWP | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9289 | T16 R06 WELS | G | 16 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9384 | E MIDDLESEX CANAL GR | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9434 | T15 R12 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9486 | T09 R11 WELS | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9518 | T10 R11 WELS | G | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9570 | T04 R09 NWP | F | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9588 | KINGSBURY PLT | E | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9608 | SPENCER BAY TWP | E | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9626 | T03 INDIAN PURCHASE TWP | F | 25 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9628 | T03 INDIAN PURCHASE TWP | F | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9668 | T05 R07 BKP WKR | E | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9670 | T05 R07 BKP WKR | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9672 | SEBOOMOOK TWP | E | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9676 | TA R11 WELS | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9678 | T01 R11 WELS | E | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9680 | T01 R11 WELS | E | 17 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9684 | T01 R12 WELS | E | 13 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9692 | T02 R09 WELS | F | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9702 | T02 R12 WELS | E | 31 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9712 | T03 R13 WELS | E | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9714 | T03 R13 WELS | E | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9718 | LOBSTER TWP | E | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9720 | T04 R12 WELS | E | 20 |  | S |  |  |  |  |  | 2 | INAC | UNDEV |  |
| UNNAMED P | 9722 | T04 R14 WELS | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9726 | T06 R13 WELS | E | 20 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9738 | DENNISTOWN PLT | E | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9740 | DENNISTOWN PLT | E | 20 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| UNNAMED P | 9742 | DENNISTOWN PLT | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9746 | ATTEAN TWP | E | 12 | m |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| UNNAMED P | 9748 | SEBOEIS PLT | F | 19 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9754 | ALDER BROOK TWP | E | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9758 | PRENTISS TWP T04 R04 NBKP | E | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9762 | DOLE BROOK TWP | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9770 | ELM STREAM TWP | E | 30 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9772 | ELM STREAM TWP | E | 25 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9774 | RUSSELL POND TWP | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9778 | T08 R19 WELS | G | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9780 | T07 R11 WELS | E | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9784 | T07 R09 WELS | F | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9786 | T07 R09 WELS | F | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9802 | T08 R10 WELS | G | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9808 | T09 R14 WELS | G | 10 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9813 | SOPER MOUNTAIN TWP | G | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9890 | T07 R08 WELS | F | 30 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9898 | T07 R08 WELS | F | 25 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9908 | T07 R07 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9920 | T06 R08 WELS | F | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UNNAMED P | 9950 | T06 R07 WELS | F | 12 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| UNNAMED P | 9972 | T05 R08 WELS | F | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| UPPER P | 1521 | T15 R09 WELS | G | 17 | O |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| VILES P | 5048 | JIM POND TWP | D | 11 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| VINING L | 1346 | CATHANCE TWP | C | 19 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| WABASSUS L | 1116 | T43 MD BPP | C | 953 | S |  |  |  |  | S |  | 2 | AC | UNDEV |  |
| WADLEIGH DEADWATER | 9091 | T01 R06 WELS | F | 58 |  |  |  |  |  |  | S | 2 | AC | UNDEV |  |
| WADLEIGH P | 0572 | T01 R11 WELS | E | 225 | S |  | S | S |  | S |  | 1B | AC | DEV |  |
| WADLEIGH P | 2011 | T04 INDIAN PURCHASE TWP | F | 97 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| WADLEIGH P | 2972 | T08 R15 WELS | E | 157 | O |  |  |  |  |  | O | 1A | AC | UNDEV |  |
| WADLEIGH P (LITTLE) | 2974 | T08 R15 WELS | E | 15 | m+ |  | + |  |  |  | O | 1B | INAC | UNDEV | 1/6 |
| WALLAGRASS (1ST&2ND) | 1628 | ST JOHN PLT | G | 281 | S |  | S |  |  |  |  | 2 | AC | DEV |  |
| WALLAGRASS L (THIRD) | 1552 | ST JOHN PLT | G | 45 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| WEBSTER L | 2718 | T06 R11 WELS | E | 531 | S | O |  |  |  |  |  | 1B | AC | UNDEV |  |
| WEBSTER P | 4678 | WEBSTER PLT | F | 40 | S |  |  |  | O |  |  | 1B | AC | UNDEV |  |
| WEEKS P | 1968 | T11 R08 WELS | G | 13 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| WEIR P | 4684 | T03 R01 NBPP | F | 45 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| WELMAN P (LOWER) | 9760 | PRENTISS TWP T04 R04 NBKP | E | 20 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| WELMAN P (UPPER) | 2482 | PRENTISS TWP T04 R04 NBKP | E | 45 | S |  |  |  |  |  |  | 2 | INAC | UNDEV | 6 |
| WEST L | 0503 | T03 ND | F | 1344 | O |  |  |  |  | S |  | 1B | AC | DEV |  |
| WEST L | 1760 | T09 R03 WELS | G | 22 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| WHEELOCK L | 1614 | ST JOHN PLT | G | 160 | S |  |  |  | O |  |  | 1B | AC | UNDEV |  |
| WHETSTONE P | 0296 | KINGSBURY PLT | E | 256 | S |  |  |  |  |  | S | 2 | AC | DEV | 5 |
| WHIPPLE P | 5094 | T05 R07 BKP WKR | E | 112 |  |  | S |  |  |  |  | 2 | AC | UNDEV |  |
| WHITE CAP P | 3960 | SEVEN PONDS TWP | D | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| WHITE P | 1462 | T13 R15 WELS | G | 17 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| WHITE HORSE L | 2162 | T07 R07 WELS | F | 218 | S |  |  |  |  |  | O | 1B | AC | UNDEV |  |
| WILLIAMS P | 0734 | T04 R11 WELS | E | 10 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| WILLIAMS P | 4486 | T28 MD | C | 19 |  |  |  |  |  |  |  | 3 | AC | DEV |  |
| WILSON HILL P | 4064 | WEST FORKS PLT | D | 18 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| WILSON HILL P (LT) | 0162 | JOHNSON MOUNTAIN TWP | E | 20 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| WILSON P (LITTLE) | 0900 | ELLIOTTSVILLE TWP | E | 30 | m |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| WILSON P (UPPER) | 0410 | BOWDOIN COL GR WEST | E | 940 | O | S | O | S |  |  | S | 1A | AC | DEV | 4 |
| WING P | 2319 | SKINNER TWP | E | 10 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| WITHEE P | 0188 | MAYFIELD TWP | D | 37 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| WITHEY BOG | 4028 | T05 R15 WELS | E | 15 |  |  |  |  |  |  |  | 3 | INAC | UNDEV |  |
| WOOD P (BIG) | 2698 | ATTEAN TWP | E | 2150 | S | S | S |  |  | S- | O | 1A | AC | DEV |  |
| WOOD P (LITTLE BIG) | 2630 | DENNISTOWN PLT | E | 713 | S | S | S | S |  |  | O | 1A | AC | UNDEV |  |
| WOODMAN P | 0622 | RAINBOW TWP | E | 6 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| WOODMAN P | 2812 | T07 R12 WELS | E | 51 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |
| WOUNDED DEER P | 2484 | PRENTISS TWP T04 R04 NBKP | E | 12 |  |  |  |  |  |  |  | 3 | INAC | UNDEV | 6 |
| WYMAN L | 7122 | PLEASANT RIDGE PLT | D | 3146 | S |  |  |  |  |  |  | 2 | AC | UNDEV |  |
| WYMAN P | 0248 | BRIGHTON PLT | D | 218 | S |  |  |  |  |  |  | 2 | INAC | UNDEV |  |
| WYTOPITLOCK L | 1702 | GLENWOOD PLT | F | 1152 | S | O |  |  |  |  |  | 1B | AC | DEV |  |
| YANKEETULADI P | 9203 | T19 R11 WELS | G | 10 |  |  |  |  | O |  |  | 1B | AC | UNDEV |  |
| YOKE PONDS | 0504 | TA R11 WELS | E | 134 | S |  |  |  |  |  |  | 2 | AC | DEV |  |
| YORK P | 3286 | GRAFTON TWP | D | 15 |  |  |  |  |  |  |  | 3 | AC | UNDEV |  |

Lakes and Rivers On Which the Use Of Personal Watercraft is Prohibited

| Lake Name | Lake # | County | Principal Town Name | Size (Acres) |
| --- | --- | --- | --- | --- |
| Allagash L | 9787 | Piscatquis | T08 R14 WELS | 4,260 |
| Alligator L | 4498 | Hancock | T34 MD | 1,159 |
| Alligator P | 0502 | Piscataquis | TA R11 WELS | 47 |
| Attean P | 2682 | Somerset | Attean Twp | 2,745 |
| Aziscohos P | 3106 | Oxford | Magalloway Twp | 12 |
| Baker P | 0422 | Piscataquis | Bowdoin Col Gr West | 10 |
| Bald Mountain P | 0314 | Somerset | Bald Mtn Twp T2R3 | 1,152 |
| Bay P (West) | 4396 | Hancock | T07 SD | 249 |
| Bean P | 0656 | Piscataquis | T02 R12 WELS | 16 |
| Bean P (Lower) | 0646 | Piscataquis | Rainbow Twp | 37 |
| Bean P (Middle) | 0648 | Piscataquis | Rainbow Twp | 10 |
| Bean P (Upper) | 0650 | Piscataquis | Rainbow Twp | 25 |
| Bear P | 0636 | Piscataquis | Rainbow Twp | 30 |
| Bear P | 4018 | Piscataquis | T06 R15 WELS | 138 |
| Beattie P | 5066 | Franklin | Beattie Twp | 27 |
| Beaver P | 3310 | Oxford | Magalloway Twp | 179 |
| Beaver P | 0670 | Piscataquis | T03 R11 WELS | 15 |
| Beaver P | 0484 | Piscataquis | Shawtown Twp | 27 |
| Beaver P (Big) | 0610 | Piscataquis | Rainbow Twp | 45 |
| Beaver P (Little) | 9700 | Piscataquis | Rainbow Twp | 8 |
| Beaver P (Little) | 0612 | Piscataquis | T03 R11 WELS | 10 |
| Benjamin P | 2684 | Somerset | Attean Twp | 121 |
| Benson P (Big) | 0864 | Piscataquis | T07 R09 NWP | 320 |
| Birch Ridge P #1 | 0514 | Piscataquis | TA R11 WELS | 11 |
| Black L | 1506 | Aroostook | T15 R09 WELS | 147 |
| Black P (Little No) | 1508 | Aroostook | T15 R09 WELS | 6 |
| Black P (Little So) | 1510 | Aroostook | T15 R09 WELS | 7 |
| Bluff P | 0434 | Piscataquis | Frenchtown Twp | 10 |
| Bluffer P (Upper) | 2798 | Piscataquis | T08 R11 WELS | 15 |
| Boardway P (Big) | 0494 | Piscataquis | TA R11 WELS | 15 |
| Bogus Meadow P | 4380 | Hancock | T07 SD | 26 |
| Boulder P | 2672 | Somerset | T05 R07 BKP WKR | 30 |
| Bowlin P (Little) | 2194 | Penobscot | T05 R07 WELS | 34 |
| Brackett P | 0290 | Piscataquis | Blanchard Plt | 10 |
| Branch P (Middle) | 0912 | Piscataquis | Ebeemee Twp | 34 |
| Branch L (South) | 2144 | Penobscot | Seboeis Plt | 2,035 |
| Brayley P | 2706 | Piscataquis | T07 R10 WELS | 6 |
| Buck P | 0644 | Piscataquis | Rainbow Twp | 6 |
| Cape Horn P | 2568 | Somerset | Prentiss Twp | 22 |
| Caribou P (Big) | 4142 | Piscataquis | T07 R10 WELS | 64 |
| Caucomgomoc L | 4012 | Piscataquis | T06 R14 WELS | 5,081 |
| Cedar P | 2654 | Somerset | HOLEB TWP | 5 |
| Cedar P | 0474 | Piscataquis | TB R10 WELS | 65 |
| Chain of Ponds | 5064 | Franklin | Chain of Ponds Twp | 700 |
| Chairback P (East) | 0802 | Piscataquis | T07 R09 NWP | 46 |
| Chairback P (West) | 0796 | Piscataquis | T07 R09 NWP | 47 |
| Chase Stream P | 4093 | Somerset | Misery Twp | 31 |
| Chesuncook L | CHCH | Piscataquis | T03 R12 WELS | 18,470 |
| Chesuncook P | 0672 | Piscataquis | T03 R11 WELS | 272 |
| Churchill L | 2856 | Piscataquis | T09 R12 WELS | 3,720 |
| Clayton P | 2406 | Somerset | T06 R17 WELS | 75 |
| Clear L | 1938 | Piscataquis | T10 R11 WELS | 614 |
| Clear P | 5074 | Franklin | Lowelltown Twp | 21 |
| Clearwater P | 2692 | Somerset | Attean Twp | 34 |
| Clearwater P | 2476 | Somerset | Prentiss Twp | 11 |
| Cliff L | 2780 | Piscataquis | T09 R12 WELS | 563 |
| Clifford L | 1304 | Washington | T27 ED BPP | 954 |
| Clifford P | 0624 | Piscataquis | Rainbow Twp | 17 |
| Clish P | 5158 | Somerset | T05 R20 WELS | 21 |
| Cranberry P (L, Notch) | 0784 | Piscataquis | Bowdoin Col Gr West | 7 |
| Crosby P | 3330 | Franklin | Coburn Gore | 150 |
| Currier P (First) | 2768 | Piscataquis | T09 R11 WELS | 20 |
| Currier P (Second) | 2774 | Piscataquis | T09 R11 WELS | 28 |
| Daisey P | 0594 | Piscataquis | T02 R10 WELS | 11 |
| Deboullie L | 1512 | Aroostook | T15 R09 WELS | 262 |
| Debsconeag | 2076 | Piscataquis | T02 R10 WELS | 500 |
| Debsconeag L (1st) | 2060 | Piscataquis | T02 R10 WELS | 320 |
| Debsconeag L (3rd) | 0584 | Piscataquis | T01 R10 WELS | 1,011 |
| Debsconeag P (6th) | 0580 | Piscataquis | T01 R11 WELS | 31 |
| Dingley P (Little) | 2462 | Somerset | T04 R05 NBKP | 17 |
| Dingley P (Upper) | 2464 | Somerset | T04 R05 NBKP | 20 |
| Dipper P | 4042 | Somerset | Pittston Acad Grant | 13 |
| Dixon P | 9911 | Somerset | Pierce Pond Twp | 17 |
| Donnell P | 4412 | Hancock | T09 SD | 1,120 |
| Doughnut P | 0616 | Piscataquis | Rainbow Twp | 12 |
| Dubois P | 2478 | Somerset | Prentiss Twp | 18 |
| Eagle L (Big) | 2858 | Piscataquis | Eagle Lake Twp | 8,288 |
| Eddy P | 3546 | Franklin | Sandy River Plt | 9 |
| Enchanted P | 0150 | Somerset | Upper Enchanted Twp | 330 |
| Enchanted P (Little) | 0148 | Somerset | Upper Enchanted Twp | 35 |
| Flagstaff L | 0038 | Somerset | Dead River Twp | 20,300 |
| Fogg P | 0426 | Piscataquis | Bowdoin Col Gr West | 23 |
| Foley P (Little) | 2492 | Somerset | Comstock Twp | 35 |
| Fowler P | 0686 | Piscataquis | T03 R11 WELS | 19 |
| Frost P (Little) | 0668 | Piscataquis | T03 R12 WELS | 35 |
| Garnder L | 1528 | Aroostook | T15 R09 WELS | 288 |
| Gauntlet P | 0472 | Piscataquis | TB R10 WELS | 11 |
| Gordon P | 0146 | Somerset | Upper Enchanted Twp | 28 |
| Gould P | 0620 | Piscataquis | Rainbow Twp | 12 |
| Great Works P | 1386 | Washington | Edmunds Twp | 50 |
| Green Mtn P | 3666 | Penobscot | T06 R06 WELS | 10 |
| Hafey P | 1498 | Aroostook | T18 R11 WELS | 23 |
| Hale P | 2508 | Somerset | Alder Brook Twp | 40 |
| Hall P | 5092 | Somerset | T05 R07 BKP WKR | 42 |
| Hall P | 2566 | Somerset | Prentiss Twp | 19 |
| Harrington P | 0702 | Piscataquis | T03 R11 WELS | 40 |
| Hathorn P | 4242 | Penobscot | T04 R08 WELS | 15 |
| Hathorn P (Little) | 2298 | Penobscot | T04 R08 WELS | 8 |
| Hedgehog P | 0556 | Piscataquis | T01 R11 WELS | 5 |
| Helen P | 0094 | Somerset | Pierce Pond Twp | 15 |
| High P | 0092 | Somerset | Pierce Pond Twp | 7 |
| Hobart Bog | 7451 | Washington | Edmunds Twp | 30 |
| Holbrook P | 0632 | Piscataquis | Rainbow Twp | 224 |
| Horserace Ponds | 0626 | Piscataquis | Rainbow Twp | 50 |
| Horseshoe P | 2686 | Somerset | Attean Twp | 50 |
| Horseshoe P | 9277 | Aroostook | T16 R09 WELS | 15 |
| Houston P (Little) | 0920 | Piscataquis | Katahdin Iron Works Twp | 27 |
| Hudson P (Upper) | 1928 | Aroostook | T11 R10 WELS | 32 |
| Hurd P (Little) | 0596 | Piscataquis | T02 R10 WELS | 60 |
| Ireland P | 4168 | Penobscot | T07 R08 WELS | 30 |
| Ironbound P | 2510 | Somerset | Alder Brook Twp | 40 |
| Jackson P #2 | 0704 | Piscataquis | T03 R11 WELS | 12 |
| Jackson P #1 | 0684 | Piscataquis | T03 R11 WELS | 23 |
| Jerry P | 2190 | Penobscot | T05 R07 WELS | 272 |
| Jim P | 5054 | Franklin | Jim Pond Twp | 320 |
| Jo-Mary L (Lower) | 0984 | Piscataquis | T01 R10 WELS | 1,910 |
| Jo-Mary L (Upper) | 0243 | Piscataquis | TA R10 WELS | 1,873 |
| Jones P | 0172 | Franklin | Wyman Twp | 36 |
| Juniper Knee P | 0878 | Piscataquis | Elliottsville Twp | 32 |
| Katahdin L | 2016 | Penobscot | T03 R08 WELS | 717 |
| Kelly P | 0654 | Piscataquis | T02 R12 WELS | 60 |
| Kennebago L (Big) | 2374 | Franklin | Davis Twp | 1,700 |
| Lane Brook P | 3664 | Penobscot | T06 R06 WELS | 33 |
| Lane P | 2490 | Somerset | Comstock Twp | 24 |
| Lang P | 2542 | Somerset | Parlin Pond Twp | 30 |
| Lang P (Little) | 2543 | Somerset | Parlin Pond Twp | 13 |
| Ledge P | 3554 | Franklin | Sandy River Plt | 6 |
| Line P | 5162 | Somerset | T05 R20 WELS | 7 |
| Lobster L | 2948 | Piscataquis | Lobster Twp | 3,475 |
| Logan P #2 | 2082 | Piscataquis | T02 R09 WELS | 20 |
| Long Bog | 2668 | Somerset | Holeb Twp | 19 |
| Long L | 1892 | Aroostook | T12 R13 WELS | 1,203 |
| Long P | 2690 | Somerset | Attean Twp | 37 |
| Long P (Little) | 4424 | Hancock | T10 SD | 55 |
| Loon P | 2688 | Somerset | Attean Twp | 37 |
| Loon P | 0554 | Piscataquis | T01 R11 WELS | 5 |
| Lost P | 2694 | Somerset | Attean Twp | 5 |
| Machias L (Third) | 1124 | Washington | T42 MD BPP | 2,778 |
| Marble P | 2186 | Penobscot | T05 R08 WELS | 75 |
| Mary Petuche P | 2474 | Somerset | Prentiss Twp | 10 |
| Mathews P | 2836 | Piscataquis | T08 R10 WELS | 19 |
| McKenna P | 0688 | Piscataquis | T03 R11 WELS | 53 |
| McKenney P | 0154 | Somerset | Upper Enchanted Twp | 9 |
| Messer P | 4244 | Penobscot | T05 R08 WELS | 27 |
| Midway P | 3544 | Franklin | Sandy River Plt | 7 |
| Millimagassett L | 3004 | Penobscot | T07 R08 WELS | 1,410 |
| Minister L (Little) | 0592 | Piscataquis | T02 R10 WELS | 4 |
| Minister P (Big) | 0590 | Piscataquis | T02 R10 WELS | 15 |
| Moccasin P | 1590 | Aroostook | T14 R08 WELS | 32 |
| Mooseleuk L | 1990 | Piscataquis | T10 R09 WELS | 422 |
| Mooselookmeguntic L | 3302 | Oxford | Richardsontown Twp | 16,300 |
| Moose P (Big) | 0334 | Piscataquis | Moosehead Junction Twp | 91 |
| Moose P (Little) | 0336 | Piscataquis | Moosehead Junction Twp | 25 |
| Mountain Catcher P | 4258 | Penobscot | T06 R08 WELS | 84 |
| Mountain P | 0432 | Piscataquis | Beaver Cove | 56 |
| Mountain View P | 0488 | Piscataquis | TA R11 WELS | 13 |
| Moxie P | 3585 | Franklin | Township D | 6 |
| Mud P | 2340 | Franklin | Township 6 N of Weld | 6 |
| Munsungan L | 4180 | Piscataquis | T08 R10 WELS | 1,415 |
| Murphy P | 0486 | Piscataquis | TA R11 WELS | 12 |
| Murphy P (Big) | 0638 | Piscataquis | Rainbow Twp | 15 |
| Muscalsea P (Big) | 4036 | Somerset | Russell Pond Twp | 14 |
| Muscalsea P (Little) | 4034 | Somerset | Russell Pond Twp | 11 |
| Musquash L (West) | 1096 | Washington | T06 R01 NBPP | 1,613 |
| Nahmakanta L | 0698 | Piscataquis | T01 R11 WELS | 1,024 |
| # Nicatous L | # 4766 | # Hancock | # T40 MD | # 5,165 |
| North P | 9781 | Aroostook | T14 R09 WELS | 15 |
| Notch P | 0786 | Piscataquis | Bowdoin Col Gr West | 10 |
| Notch P (Big) | 0328 | Piscataquis | Moosehead Junction Twp | 12 |
| Notch P (Little) | 0326 | Piscataquis | Moosehead Junction Twp | 10 |
| Papoose P | 0338 | Piscataquis | Moosehead Junction Twp | 3 |
| Passamagamet L | 0970 | Piscataquis | T01 R09 WELS | 461 |
| Penobscot L | 0339 | Somerset | Dole Brook Twp | 1,019 |
| Pierce P | 0086 | Somerset | Pierce Pond Twp | 1,650 |
| Pitman P | 0598 | Piscataquis | T02 R10 WELS | 20 |
| Pleasant L | 1100 | Washington | T06 R01 NBPP | 1,574 |
| Poland P (Upper) | PPUP | Piscataquis | T07 R14 WELS | 245 |
| Polly P | 0692 | Piscataquis | T03 R11 WELS | 15 |
| Porter P | 4760 | Hancock | T03 ND | 58 |
| Rabbit P | 0366 | Piscataquis | Elliottsville Twp | 10 |
| Rabbit P | 0552 | Piscataquis | T01 R11 WELS | 10 |
| Rainbow Deadwaters | 9698 | Piscataquis | Rainbow Twp | 58 |
| Rainbow L | 0614 | Piscataquis | Rainbow Twp | 1,664 |
| Rainbow P | 4436 | Hancock | T10 SD | 17 |
| Reed P (Big) | 2842 | Piscataquis | T08 R10 WELS | 90 |
| Reed P (Little) | 2838 | Piscataquis | T08 R10 WELS | 25 |
| Ripogenus P | 2910 | Piscataquis | T04 R12 WELS | 76 |
| Roach P (Fourth) | 0446 | Piscataquis | Shawtown Twp | 266 |
| Roach P (Seventh) | 0500 | Piscataquis | TA R11 WELS | 33 |
| Roach P (Sixth) | 0480 | Piscataquis | Shawtown Twp | 48 |
| Robar P (Big) | 2296 | Penobscot | T04 R08 WELS | 7 |
| Roberts P | 5164 | Somerset | T05 R20 WELS | 19 |
| Rocky P (Little) | 0524 | Piscataquis | TA R11 WELS | 12 |
| Round P | 2670 | Somerset | Appleton Twp | 5 |
| Round P | 1470 | Aroostook | T13 R12 WELS | 697 |
| Round P (Little) | 2874 | Piscataquis | Eagle Lake Twp | 58 |
| Saddleback P | 3550 | Franklin | Sandy River Plt | 13 |
| Sawtelle P | 3008 | Penobscot | T07 R08 WELS | 174 |
| Sawtelle P (Little) | 5778 | Penobscot | T07 R08 WELS | 10 |
| Scraggly L | 4264 | Penobscot | T07 R08 WELS | 842 |
| Secret P | 0907 | Piscataquis | Elliottsville Twp | 12 |
| Slaughter P | 0690 | Piscataquis | T03 R11 WELS | 66 |
| Snake P | 2548 | Somerset | Johnson Mountain Twp | 8 |
| Socatean P #1 | 4044 | Somerset | Plymouth Twp | 42 |
| Socatean P #2 | 4046 | Somerset | Plymouth Twp | 14 |
| Speck P | 3288 | Oxford | Grafton Twp | 9 |
| Spencer L | 5104 | Somerset | Hobbstown Twp | 1,819 |
| Spencer P | 0404 | Piscataquis | E Middlesex Canal Gr | 980 |
| # Spring L | # 0170 | # Somerset | # T03 R04 BKP WKR | # 762 |
| Spring P | 2832 | Piscataquis | T07 R10 WELS | 15 |
| Spruce Mountain P | 0466 | Piscataquis | TB R11 WELS | 20 |
| St. John P (Second) | 2432 | Somerset | T04 R17 WELS | 105 |
| St. John P (Third) | 2438 | Somerset | T04 R17 WELS | 190 |
| St. John P (Lower 1st) | 2428 | Somerset | T04 R17 WELS | 29 |
| St. John P (Upper 1st) | 2440 | Somerset | T04 R17 WELS | 30 |
| Stratton P | 0618 | Piscataquis | Rainbow Twp | 15 |
| Sunday P | 3316 | Oxford | Magalloway Twp | 30 |
| Swift River P (Little) | 3572 | Franklin | Township E | 15 |
| Telos L & Round P | 2710 | Piscataquis | T06 R11 WELS | 2,276 |
| The Horns P | 8601 | Franklin | Wyman Twp | 10 |
| Tilden P | 4418 | Hancock | T10 SD | 36 |
| Tim P | 2362 | Franklin | Tim Pond Twp | 320 |
| Tobey P #1 | 2674 | Somerset | T05 R07 BKP WKR | 35 |
| Tobey P #2 | 2676 | Somerset | T05 R07 BKP WKR | 32 |
| Tobey P #3 | 2678 | Somerset | T05 R07 BKP WKR | 14 |
| Trout L | 1098 | Washington | Kossuth Twp | 5 |
| Trout P | 5082 | Franklin | Lowelltown Twp | 55 |
| Trout P | 3260 | Oxford | Mason Twp | 17 |
| Trout P | 0792 | Piscataquis | Bowdoin Col Gr West | 20 |
| Tumbledown Dick P | 0548 | Piscataquis | T01 R11 WELS | 24 |
| Tumblwdown P | 3512 | Franklin | Township 6 N of Weld | 9 |
| Tunk L | 4434 | Hancock | T10 SD | 2,010 |
| Turtle P | 0952 | Piscataquis | Lake View Plt | 81 |
| Twin (Trout) Ponds | 2102 | Piscataquis | T02 R09 WELS | 60 |
| Two Mile P | 9765 | Aroostook | T16 R13 WELS | 12 |
| Umsaskis L | 1896 | Aroostook | T11 R13 WELS | 1,222 |
| Unnamed P | 9746 | Somerset | Attean Twp | 12 |
| Unnamed P | 8980 | Somerset | T05 R07 BKP WKR | 10 |
| Unnamed P | 8942 | Somerset | Holeb Twp | 2 |
| Unnamed P | 8934 | Somerset | Attean Twp | 5 |
| Unnamed P | 8416 | Somerset | Comstock Twp | 20 |
| Unnamed P | 7073 | Piscataquis | T06 R15 WELS | 8 |
| Unnamed P | 7115 | Somerset | Comstock Twp | 15 |
| Unnamed P (Fernald) | 8868 | Somerset | Parlin Pond Twp | 7 |
| Upper Wilson P | 0410 | Piscataquis | Bowdoin Col Gr West | 940 |
| Wadleigh P (Little) | 2974 | Piscataquis | T08 R15 WELS | 15 |
| Welman P (Upper) | 2482 | Somerset | Prentiss Twp | 45 |
| Wing P | 2319 | Franklin | Skinner Twp | 10 |
| Woodman P | 0622 | Piscataquis | Rainbow Twp | 6 |
| Wounded Deer P | 2484 | Somerset | Prentiss Twp | 12 |
| River Name |  | County | Town Names |  |
| Kennebago |  | Franklin | Davis and Stetsontown |  |

(RESERVED)

EXPEDITED PERMITTING AREA FOR WIND ENERGY DEVELOPMENT

The following areas, not including areas below the high water mark of coastal wetlands, comprise the expedited permitting area for wind energy development to accomplish the purposes of “An Act to Implement Recommendations of the Governor’s Task Force on Wind Power Development”, P.L. 2007,
ch. 661, and “An Act To Improve Regulatory Consistency within the Jurisdiction of the Maine Land Use Planning Commission,” P.L. 2015, ch. 265. The Commission may add and remove areas within its jurisdiction to and from the expedited permitting area for wind energy development in accordance with Title 35-A, Section 3453 and Section 3453-A.

Entire townships and plantations.  The following entire townships and plantations:

Alder Stream Twp., 07801;

Argyle Twp., 19801;

Bald Mountain Twp., T2 R3, 25805;

Baring Plt., 29040;

Barnard Twp., 21030;

Batchelders Grant Twp., 17805;

Benedicta Twp., 03050;

Berry Twp, 29818;

Blake Gore, 25811;

Blanchard Twp., 21040;

Brookton Twp., 29801;

Centerville Twp., 29080;

Chase Stream Twp., 25816;

Chester, 19100;

Connor Twp., 03802;

Cox Patent, 03803;

Cross Lake Twp., 03899;

Cyr Plt., 03140;

Dudley Twp., 03804;

Dyer Twp., 29803;

E Twp., 03160;

East Moxie Twp., 25821;

Fletchers Landing Twp., 09804;

Forkstown Twp., 03805;

Fowler Twp., 29807;

Garfield Plt., 03220;

Glenwood Plt., 03230;

Grand Falls Twp., 19250;

Grindstone Twp., 19802;

Hamlin, 03250;

Hammond, 03260;

Herseytown Twp., 19803;

Hibberts Gore, 15801;

Hopkins Academy Grant Twp., 19804;

Indian Stream Twp., 25828;

Jim Pond Twp., 07811;

Johnson Mountain Twp., 25829;

Kibby Twp., 07812;

Kingman Twp., 19808;

Lake View Plt., 21120;

Macwahoc Plt., 03360;

Madawaska Lake Twp, 03889;

Mattamiscontis Twp., 19810;

Mayfield Twp., 25835;

Milton Twp., 17812;

Misery Twp., 25836;

Moro Plt., 03430;

Mount Chase, 19450;

Nashville Plt., 03440;

North Yarmouth Academy Grant Twp., 03807;

Osborn, 09230;

Oxbow North Twp., 03500;

Parkertown Twp., 17814;

Perkins Twp., 07818;

Perkins Twp. Swan Island, 23801;

Prentiss Twp., T4 R4 NBKP, 25843;

Prentiss Twp., T7 R3 NBPP, 19540;

Reed Plt., 03540;

Saint Croix Twp., 03808;

Saint John Plt., 03570;

Sandy Bay Twp., 25850;

Sandy River Plt., 07170;

Scopan Twp., 03810;

Seboeis Plt., 19550;

Silver Ridge Twp., 03809;

Sinclair Twp., 03898;

Squaretown Twp., 25854;

Summit Twp., 19812;

T1 R5 WELS, 03816;

T1 R6 WELS, 19815;

T10 R3 WELS, 03829;

T10 R6 WELS, 03830;

T10 SD, 09806;

T11 R3 NBPP, 29817;

T11 R4 WELS, 03833;

T13 R5 WELS, 03856;

T14 R5 WELS, 03867;

T14 R6 WELS, 03868;

T15 R5 WELS, 03879;

T15 R6 WELS, 03880;

T16 MD, 09807;

T16 R5 WELS, 03890;

T16 R6 WELS, 03891;

T18 MD BPP, 29819;

T19 ED BPP, 29820;

T19 MD BPP, 29821;

T2 R4 WELS, 03817;

T2 R8 NWP, 19817;

T2 R9 NWP, 19819;

T22 MD, 09808;

T3 Indian Purchase Twp., 19806;

T3 R3 WELS, 03818;

T3 R4 WELS, 03819;

T3 R9 NWP, 19823;

T4 R3 WELS, 03820;

T6 R6 WELS, 19829;

T7 R5 WELS, 03821;

T7 R6 WELS, 19832;

T8 R3 NBPP, 29815;

T8 R3 WELS, 03822;

T8 R4 NBPP, 29816;

T8 R5 WELS, 03823;

T8 R6 WELS, 19835;

T9 R3 WELS, 03824;

T9 R4 WELS, 03825;

T9 R5 WELS, 03826;

T9 SD, 09805;

TA R2 WELS, 03813;

TA R7 WELS, 19814;

TC R2 WELS, 03814;

TD R2 WELS, 03815;

Unity Twp., 11801;

Upper Molunkus Twp., 03811;

Van Buren Cove Twp., 03897;

Washington Twp., 07827;

Webbertown Twp., 03812;

Webster Plt., 19600;

Williamsburg Twp., 21827;

and Winterville Plt., 03680;

Portions of townships and plantations.  The following portions of townships and plantations:

that portion of Adamstown Twp., 17801, north of Route 16;

Bald Mountain Twp., T4 R3, 25806, excluding areas of Boundary Bald Mountain above 2,700 feet in elevation;

Chain of Ponds, 07803, an approximately 1,578.4-acre parcel, bounded by the eastern town line at latitude 45.373, longitude -70.625484, proceeding westerly to latitude 45.370087, longitude
-70.63231 then to latitude 45.368156, longitude -70.645478 where it intersects the 2,400-foot contour, proceeding along the 2,400-foot contour westerly and then northerly to the intersection of the 2,400-foot contour to the northern town line, following the town line eastward and then southward until reaching the beginning point, latitude 45.373, longitude -70.625484;

the portion of Coplin Plt., 07040, north of Route 16;

the portion of Dallas Plt., 07050, north of Route 16;

the portion of Ebeemee Twp., 21853, east of Route 11;

the portion of Kossuth Twp., 29808, north of Route 6 and a portion south of Route 6, beginning at a point 45.406587° latitude, -67.97342° longitude on the boundary between Carroll Plantation and Kossuth Township approximately 1,500 feet southeast of Route 6 along the township line, and following the town boundary southeast approximately 4,000 feet to 45.39584° latitude,
-67.970263° longitude, then proceeding east approximately 7,500’ to 45.395571° latitude,
-67.940835° longitude, then proceeding north approximately 3,200 feet to 45.404206° latitude, -67.940674° longitude, then proceeding southwest approximately 935 feet to 45.403537° latitude, -67.944191° longitude, then proceeding north approximately 1,000 feet to 45.406328° latitude, -67.944987° longitude, then proceeding east approximately 7,300 feet to the point of beginning, 45.406587° latitude, -67.97342° longitude;

the portion of Lang Twp., 07813, north of Route 16;

the portion of Lincoln Plt., 17160, north of Route 16;

the portion of Long A Twp., 19809, east of Route 11;

the 487.5-acre area above the 2,040-foot elevation around Green Top in Lynchtown Twp., 17810;

the portions of Misery Gore Twp., 25837, north of and administered as part of Misery Twp. and Parlin Pond Twp. pursuant to P & S.L. 1991, ch. 51, §2;

the portion of Molunkus Twp., 03806, beginning at a point on the line between Molunkus (a.k.a. Township A Range 5 WELS) and Macwahoc Plantation being located S 1º 40’ 53” E, a tie distance of 208.77 feet from an iron rod located at coordinates N: 16,578,625.02 usft  E: 1,819,527.31 usft; Thence S 71º 39’ 16” W, a distance of 3,783.03 feet to a point on the easterly sideline of the Benedicta Road, so called; Thence continuing S 71º 39’ 16” W, crossing said Benedicta Road a distance of 283.96 feet to a point; Thence S 14º 04’ 18” W, a distance of 4,456.43 feet; Thence S 44º 41’ 42” W, a distance of 15,391.59 feet; Thence S 47º 01’ 09” W, a distance of 1,202.86 feet to an angle point having coordinates N: 16,561,052.23 usft  E: 1,802,884.24 usft; Thence S 63º 11’ 34” W, a distance of 2,580 feet, more or less, to a point on the southerly line of Molunkus and the southerly line of Aroostook County; Thence S 77º 00’ W, more or less, by and along said southerly line of Molunkus, a distance of 840 feet, more or less, to a point located 200.00 feet from, as measured perpendicular to, the  last described course; Thence N 63º 11’ 34” E, at all times 200.00 feet northwesterly of and parallel to, the southeasterly line of the parcel described herein, a distance of 3365 feet, more or less, to a point having coordinates N: 16,561,217.92 usft  E: 1,802,768.68 usft; Thence N 47º 01’ 09” E, at all times 200.00 feet northwesterly of and parallel, a distance of 1,170.39 feet to a point; Thence N 44º 41’ 42” E, at all times 200.00 feet northwesterly of and parallel to, the southeasterly line of the parcel described herein, a distance of 15332.78 feet to a point; Thence N 14º 04’ 18” E, at all times 200.00 feet northwesterly of and parallel to, the southeasterly line of the parcel described herein, a distance of 4,511.59 feet to a point; Thence N 71º 39’ 16” E crossing said Benedicta Road, at all times 200.00 feet northwesterly of and parallel to, the southeasterly line of the parcel described herein, a distance of 338.62 feet to an iron rod found on the easterly sideline of said Benedicta Road; Thence continuing N 71º 39’ 16” E, a distance of 3,898.14 feet to said iron rod on the line between Molunkus and Macwahoc Plantation; Thence S 1º 40’ E, more or less, by and along said line between Molunkus and Macwahoc Plantation, a distance of 209 feet, more or less, to the point of beginning;

the portion of Rockwood Strip T2 R1 NBKP, 25845, south of Little Brassua Lake and Brassua Lake;

the portion of Sandwich Academy Grant Twp., 25849, south of Moose River, Little Brassua Lake and Brassua Lake;

Skinner Twp., 07822, an approximately 193.4-acre parcel that follows the ridge to Kibby Mountain, bounded on the east and west by the 2,820-foot contour, on the south by the town line where it intersects the 2,820-foot contour and on the north by a line drawn from latitude 45.4121, longitude -70.54402 to latitude 45.41587, longitude -70.5349 intersecting the 2,820-foot contour;

the portion of Soldiertown Twp., T2 R7 WELS, 19811, east of the East Branch Penobscot River;

the portion of T1 R8 WELS, 19816, south of Millinocket Lake;

the portion of T1 R9 WELS, 21833, southeast of Ambajejus Lake;

T24 MD BPP, 29822, excluding a one mile buffer around Mopang Stream;

T25 MD BPP, 29823, an approximately 558.5-acre parcel in the Bear Brook and Black Pond area, bounded by a point along the southern town line, latitude 44.805142, longitude -67.741067, and proceeding in a counterclockwise direction through the following points, latitude 44.808871, longitude -67.744217, latitude 44.812645, longitude -67.750877, latitude 44.816887, longitude -67.76346, latitude 44.817639, longitude -67.768806, latitude 44.817596, longitude
-67.770188, latitude 44.817259, longitude -67.771089, latitude 44.816282, longitude
-67.771687, latitude 44.815068, longitude -67.771704, latitude 44.810286, longitude
-67.767988, latitude 44.802482, longitude -67.759738 intersecting the town line, proceeding easterly along the southern town line to beginning point latitude 44.805142, longitude
-67.741067;

the portion of T3 R7 WELS, 19821, east of the Seboeis River and East Branch Penobscot River;

the portions of T4 Indian Purchase Twp., 19807, area northeast of North Twin Lake and south of Route 11;

the portion of T4 R7 WELS, 19824, east of the Seboeis River;

the portion of T4 R9 NWP, 21845, east of Route 11;

the portion of T5 R7 WELS, 19827, east of the Seboeis River; and

the portion of T6 R7 WELS, 19830, east of the Seboeis River; and

Coastal islands in unorganized and deorganized area.  All islands located in waters subject to tidal influence that are within the unorganized and deorganized areas of the State.

- Appendix G	Designated areas of Cultural or Special Significance (DacSS)

    - Designated Area of Cultural or Special Significance (DACSS).  Areas of cultural or special significance may be designated by the Commission to facilitate expansion, reconstruction or replacement, or relocation of structures that provide significant public benefits to the surrounding community. A DACSS is intended to include discrete geographic areas with significant cultural or other special features. Once included in a DACSS, a structure may be expanded in accordance with Section 10.11,C,1,c, reconstructed or replaced in accordance with Section 10.11,C,2,f, and relocated in accordance with Section 10.11,C,3, of the Commission’s rules. Designation of an area as a DACSS does not preclude expansion, reconstruction or replacement, or relocation of legally existing non-conforming structures in accordance with other applicable provisions of Section 10.11.

        - Designation Criteria.  The Commission must designate an area of cultural or special significance upon finding that a proposed area meets the following criteria:

          - The proposed area is an iconic location containing features that convey a sense of time or place to residents or visitors;

          - The proposed area is a geographically definable area or neighborhood composed of a finite group of related features that have a special character, historical interest, or aesthetic value; and

          - The proposed area provides a significant public benefit, per Section 1,b.

        - Contributing Factors.  When evaluating whether an area proposed for designation provides a significant public benefit, as required by Appendix G, Section 1,c the Commission must consider the factors listed below. The Commission may find an area provides a significant public benefit even if the area does not include features satisfying all of the factors set forth in subsections (1) through (4) below, provided the evidence offered for a particular factor or subset of factors is compelling. In applying these factors, the Commission may consider, among other evidence, documentation such as published books, magazines, academic writings, or newspaper articles; other historic records identifying the value of the feature or area; and visual depictions such as postcards, images or paintings. The Commission also may receive testimony by persons with special knowledge about applicable factors.

          - Cultural:  Buildings, improvements or other features that exemplify or reflect a group of people linked together by shared values, traditional uses, beliefs, and historical associations;

          - Historic:  Represents a distinct historical period, type, style, region, or way of life; is the site of an important historical event, or is associated with historic events that have made a meaningful contribution to the nation, state, or community; or represents a specific business or use, which either once was common but now is rare, or continues to have historical value as it currently operates.

          - Architectural or Engineering:  Exemplifies a particular architectural style or way of life, construction materials or engineering methods used to build features, or embodies elements of outstanding attention to architectural or engineering design, detail, material, or craftsmanship.

          - Neighborhood or Geographic Setting:  The proposed area is in a unique location, or represents an established and familiar visual feature of the community that draws residents and visitors, or structures or features within the designated area include singular physical characteristics.

        - Procedural Requirements.  In accordance with Chapter 4 of the Commission’s Rules, Rules of Practice, Section 4.05,G, any person may petition the Commission for designation of an area as a DACSS. Petitions must be in writing and identify the area proposed for designation, specify the purpose of the proposed designation, and explain how the designation conforms with the criteria set forth in Appendix G, Section1,a. In addition to notification requirements described in Section 4.04,B,2 upon receiving a petition to designate a proposed area the Commission must notify:  (i) property owners within the area proposed as a DACSS; (ii) the municipality or plantation where the designated area is proposed; (iii) the county if located in an unorganized township; (iv) appropriate state and federal agencies, to be determined by the Commission; and (v) tribal governments. Upon designation of an area as a DACSS, a description of the designated area, its purpose, and the facts substantiating the Commission’s finding that the applicant satisfied the criteria in Appendix G, Section 1,a must be listed in Appendix G, Section 2 of this chapter.

        - Removal of Designation.  Upon petition in accordance with Chapter 4 of the Commission’s Rules, Rules of Practice, Section 4.05,G, the Commission may remove a designation of a DACSS when the Commission finds that the DACSS no longer meets the purpose of the DACSS listed in Appendix G, Section 2, of this chapter or no longer provides a significant public benefit based on the factors in Appendix G, Sections 1,b,(1) through (4).

    - Designated Areas.  The following areas have been designated as a DACSS in accordance with the procedure described in Appendix G, Section 1.

        - Grand Lake Stream Plantation Boathouses and Waterfront. Grand Lake Stream Plantation is a place where fishing, boating, access to water, and a strong connection to the past are key components of its identity and culture and very important to its residents, as well as the large number of visitors that come to the community each year. It is a place to which people from all over the world travel for its culture, its beauty, and importantly, the "old-time feeling" that makes it special.

          - Designation Purpose. The purpose of listing the Grand Lake Stream shorefront as a designated area is to provide the community an opportunity to sustain the boathouses located on the iconic southern shoreline of West Grand Lake that symbolize key components of its culture, and history, and that convey a tranquil "old-time feeling" for the people that live in and visit the community, and contribute to Grand Lake Stream Plantation’s quality of life and sense of place.

          - Geographic Boundaries. The designated area includes all structures within its boundaries that are legally existing as of the effective date of this rule amendment. The designated area specifically includes the two public boat launches and excludes the man-made dam and any portion thereof, and is composed of the following two sub-areas:

            - Western sub-area. Extends northerly and westerly from the intersection of the driveway for the public boat launch and centerline of Shaw Road, along the centerline of Shaw Road, to the point where Shaw Road crosses the inlet into Hatchery Cove; and extends on the lake side of the road, to the extent necessary to include all legally existing structures.

            - Eastern sub-area. Extends southerly from the end of Canal Street, along the centerline of Canal Street, to the center of the intersection of Canal Street and the driveway leading to the dam; and extends  on the lake side of the road, to the extent necessary to include all legally existing structures.

          - Facts Substantiating Designation of Grand Lake Stream Plantation Boathouses and Waterfront as a Designated Area of Cultural or Special Significance.

            - The historic boat houses that have existed along the shoreline at the southern end of West Grand Lake since the early 1900's are part of what makes the area described in Appendix G, Section 2,a iconic. For residents and visitors, the boathouses reflect the community, its culture, and their shared value of traditional uses and an "old-time feeling."

            - The boathouses in Hatchery Cove are representative of the Maine Guiding Tradition and a specific way of life. They originally sheltered steamboats used to transport sportsmen and other visitors to remote lodges located around the lake. They have since been used to store boats for visiting sportsmen, including the famous Grand Laker Canoe, which was designed and built locally for conditions commonly found on the lake.  Additionally, the boathouses have been used by property and business owners to supply camps and lodges located around the lake, and by the Maine Forest Service as a base for firefighting operations. Although privately owned and used, the community associates the boathouses with their past.  One resident commented that all good stories start with “we were down at the boathouses ….”

            - The area described in Appendix G, Section 2,a,(1) is a unique geographic location and represents an established and familiar visual feature of the community that draws residents and visitors, and contributes to a high quality of life for residents and economic activity from tourism. Residents talk of children in the area, against the wishes of their parents, jumping from the boathouses into the lake.  Fishermen visit the area to check the flow in the river and to launch their boats on the lake. Hikers, ATV riders, paddlers and snowmobilers use the area to access the many nearby recreational trails. Families and school groups come to swim and picnic. The community holds events at the waterfront such as fireworks displays and activities during the annual Grand Lake Stream Folk Arts Festival. Photographers have a single place where they can capture many of the important elements that contribute to the culture and sense of place that locals and visitors experience in the community. Residents describe this area as Grand Lake Stream’s “working waterfront,” and as the “core of the town.”